FINAL

ARP Pilot Area 4 Climatological Report: Port Hawkesbury and the Strait of Canso

Submitted to:

Environment and Climate Change Canada

Attn: Xiaolan L. Wang Climate Research Division 4905 Dufferin Street, Room 4N807 Toronto, ON, M3H 5T4 Canada

Submitted by:

Amec Foster Wheeler Environment & Infrastructure,

a Division of Amec Foster Wheeler Americas Limited

133 Crosbie Road PO Box 13216 St. John’s, NL A1B 4A5 Canada

31 March 2017

Amec Foster Wheeler Project #: TA1664202

IMPORTANT NOTICE

This report was prepared exclusively for Environment and Climate Change Canada by Amec Foster Wheeler Environment & Infrastructure, a Division of Amec Foster Wheeler Americas Limited (Amec Foster Wheeler). The quality of information, conclusions and estimates contained herein is consistent with the level of effort involved in Amec Foster Wheeler’s services and based on: i) information available at the time of preparation, ii) data supplied by outside sources and iii) the assumptions, conditions and qualifications set forth in this report. This report is intended to be used by Environment and Climate Change Canada only, subject to the terms and conditions of its contract with Amec Foster Wheeler. Any other use of, or reliance on, this report by any third party is at that party’s sole risk.

EXECUTIVE SUMMARY

This study focuses on the recent climatology and projected effects of climate change on wind and waves in Area Response Plan (ARP) Pilot Area 4: Port Hawkesbury and the Strait of Canso. This report is one of four produced during this study, each of which focuses on a different ARP Pilot Area in Canadian coastal waters, all of which were selected due to their high volume of marine traffic. This project supports the Government of Canada’s ARP Initiative which aims to strengthen national marine oil spill preparedness and response. The purpose of this report, and the study on which it is based, is to provide “operational science to inform decision making for effective emergency preparedness and response.”

Climatological analysis components included the calculation of various descriptive statistics for significant wave heights and wind speeds as well as numerous extreme value indices, defined by Environment and Climate Change Canada (ECCC). Spatial statistics and indices were determined for each month, season and year. Long-term means and linear trends were then determined for all calculated values. Results of all means and trends were then mapped. Analysis and maps were also produced for changes in mean wind direction for each month.

Climate change analysis was conducted using an ensemble of latest generation Coupled Model Intercomparison Project (CMIP5) global climate model (GCM) output. The climate change signal for all statistics, indices and wind directions mentioned above were calculated for two greenhouse gas emissions scenarios (RCP 4.5 and RCP 8.5) and two planning horizons (mid and end 21st century). Minimum, median and maximum ensemble projection results were then mapped to ensure a broad range of climate projection uncertainty was represented.

It needs to be stressed that only four coarse resolution CMIP5 GCMs were used in the study. The 1° grid resolution for waves resulted in poor wave data coverage in areas directly adjacent to coastlines. This relatively small ensemble size and coarse resolution leads to large uncertainties in the projected results. All accompanying discussion herein refers to only those grid-points shown in the maps and results should be interpreted through this lens.

The long-term mean of minimum annual wave heights were less than 0.4 m, median annual wave heights were between 0.5 and 1.75 m, and maximum annual wave heights ranged from 3.0 to 9.0 m. Significant wave height values were generally found to increase with distance from the coastline. Minimum, median and maximum wave heights have all been found to be trending slightly upward over the past several decades. There was a broad range of frequency of rough wave days from 2% of days per year in sheltered coastal areas to 45% of days further offshore. The frequency of high wave days ranged from 0 to 3%. The number of consecutive days with top-decile wave heights was between 26 and 29 days. All of these indices have been trending upward in various parts of the region.

According to the ensemble mean climate change projections, wave heights are generally expected to decrease moderately in the coming decades, which runs counter to the historical trends. The frequency of rough and high wave days are projected to decrease, while the number of days with consecutive top-decile wave days is projected to increase slightly.

While the current sea states in the area present favourable wave conditions for vessel navigation, manoeuvring and marine operations, the ensemble maximum projected changes in winter show increased wave heights in the region. This would increase the risk of shipping delays and potential damage for marine operations as the wave heights may exceed operational limits. This would result in decreasing time for ship manoeuvring and loading/unloading at terminal and increasing berthing time at terminal and delaying departure. Marine facilities and coastal infrastructure may also be subject to greater damage from higher winter waves. Projected changes are unlikely to impact significantly on marine activities in Port Hawkesbury and the Strait of Canso in spring, summer and fall.

With regard to spill response capability, projected changes in wave conditions and the frequency of rough wave days are generally unlikely to measurably affect spill response planning or success and suggest no measurable change to the windows of opportunity for conducting spill response.

The long-term mean of minimum annual wind speeds were 0.2 to 0.5 m/s, the median annual wind speeds were around 7.0 to 8.0 m/s, and the maximum annual wind speed ranged from 22.0 to 25.0 m/s. While the minimum wind speeds were trending slightly upwards in some areas, the median and maximum wind speeds were experiencing slightly negative or positive trends, depending on the specific location. Long-term means of high wind indices, which measured frequencies of days with wind speeds greater than 14.4 m/s up to the frequency of days with wind speeds greater than 24.7 m/s. These ranged from 14.5-21.5% down to 0.025-0.15%, respectively. Consecutive days experiencing top-decile and bottom-decile wind speeds were 25.0-26.0 and 22.0-22.5 days, respectively. No significant trends were found for any of the extreme wind indices.

The long-term mean wind direction for each month was typically between east-south-eastward in winter and north-eastward in summer. Relatively small shifts (from 1980-1995 to 2000-2015) in wind direction (between 14° clockwise and anti-clockwise) were found to occur in most months. April showed clockwise changes closer to shore (up to 21°) and anti-clockwise changes further offshore (up to 14°). May exhibited the largest changes with clockwise rotation of up to 35° in offshore areas.

The ensemble mean projections show only marginal changes for minimum and median wind speeds, while the maximum wind speed is projected to decrease by 1.5 m/s. The frequencies of days with high winds (i.e. greater than 14.4, 17.2, and 20.8 m/s) are projected to increase, with virtually no change in the number of calm days or those with storm-force winds. The number of consecutive days experiencing top- or bottom-decile wind speeds is projected to increase or decrease, depending on location.

Winter winds are a key risk factor for marine transportation in the region as wind speeds may exceed the operational limits and result in shipping delays and vessel damage. Ensemble maximum projections in winter wind speeds could lead to more downtime for marine transportation and produce higher risk of vessel damage. Projected changes in monthly wind direction mostly occur in summer meaning the impact on marine transportation due to wind direction changes will not likely be an issue.

With regard to spill response, average historical wind conditions indicate the effect of spill drift due to wind would be towards offshore. For those few months with projected changes in wind direction, the tendency is for increased frequency of winds more from the southwest (i.e., northeastward). In the case of an oil spill this might mean increased likelihood of drift tangential to the Nova Scotia shoreline. None of the projected changes in wind speed would have a bearing on spill response.

Projected increases in temperature may lead to more frequent fog events. This decreased visibility would increase navigation risk for marine transportation in this area. Rising sea-levels may results in accelerated erosion and accelerated wear on coastal infrastructure. Deeper water in harbours, however, could present new opportunities by improving access for larger ships with deeper drafts.

In terms of oil spill response, warmer conditions will contribute to decreased viscosity (and hence increased spreading) and an increased rate and degree of evaporation. The projected decrease in the presence of sea ice, however, could remove several complicating factors for spill response.  Surface currents will continue to play a significant role in the drift of an oil spill, with slicks drifting with the same speed and direction as the current.

Table of Contents

EXECUTIVE SUMMARY	3

1.0	Introduction	11

1.1	Report Organization	12

2.0	Wind and Wave Climatological Analysis	13

2.1	Summary Statistics	13

2.2	Indices	13

2.3	Description of Datasets and Models	15

2.3.1	MSC50 Atlantic	15

2.3.2	NCEP CFSR	15

2.3.3	CMIP5 Global Climate Models	16

3.0	Historical Wind and Waves Climate and Trend Analysis	17

3.1	Minimum Wave Heights	18

3.2	Maximum Wave Heights	21

3.3	Extreme Wave Height Indices	24

3.4	Minimum Wind Speeds	27

3.5	Maximum Wind Speeds	30

3.6	Extreme Wind Speed Indices	33

3.7	Monthly Wind Direction	38

4.0	Projected Future Changes based on four CMIP5 Models	44

4.1	Minimum Wave Heights	45

4.2	Maximum Wave Heights	51

4.3	Extreme Wave Height Indices	57

4.4	Minimum Wind Speeds	64

4.5	Maximum Wind Speeds	70

4.6	Extreme Wind Speed Indices	76

4.7	Monthly Wind Direction	87

4.8	Evaluation of Simulated Wind from Climate Models	93

5.0	Discussion of Analysis Results	95

5.1	Key Risks and Impacts for the Marine Transportation Industry	95

5.1.1	Significant Wave Height	95

5.1.2	Surface Wind Speed and Direction	96

5.2	Key Risks and Impacts for Oil Spill Response	97

5.2.1	Significant Wave Height	98

5.2.2	Surface Wind Speed and Direction	100

5.3	Potential Impact of other Marine Climate Elements	102

5.3.1	Sea Surface Temperature and Air Temperature	102

5.3.2	Sea Ice and Icebergs	104

5.3.3	Fog	106

5.3.4	Sea-Level Rise, Storm Surge and Currents	106

6.0	Summary	108

7.0	Signatures	112

8.0	REFERENCES	113

List of Tables

Table 2-1: Extreme Wave Height Indices	13

Table 2-2: Extreme Wind Indices	14

Table 2-3: CMIP5 Model Information	16

Table 3-1: Historical Significant Wave Height Statistics Summary	17

Table 3-2: Historical Surface Wind Speed Statistics Summary	17

Table 3-3: Historical Extreme Wave Height Indices Summary	24

Table 3-4: Historical Extreme Wind Speed Indices Summary	33

Table 4-1: Projected Changes in Minimum Wave Height Summary	45

Table 4-2: Projected Changes in Maximum Wave Height Summary	51

Table 4-3: Projected Changes in Extreme Wave Height Indices Summary	57

Table 4-4: Projected Changes in Minimum Wind Speed Summary	64

Table 4-5: Projected Changes in Maximum Wind Speed Summary	70

Table 4-6: Projected Changes in Extreme Wind Speed Indices Summary	77

Table 6-1: Historical Annual Statistics Summary	108

Table 6-2: Historical Extreme Wave Height Indices Summary (from Table 3-3)	109

Table 6-3: Historical Extreme Wind Speed Indices Summary (from Table 3-4)	109

Table 6-4: Ensemble Mean Projected Changes in Annual Statistics Summary	110

Table 6-5: Ensemble Mean Projected Changes in Extreme Wave Height Indices Summary	110

Table 6-6: Ensemble Mean Projected Changes in Extreme Wind Speed Indices Summary	110

list of Figures

Figure 1-1 Area Response Plans Pilot Areas (ECCC)	11

Figure 3-1 Annual minimum significant wave height.	18

Figure 3-2 Winter minimum significant wave height.	19

Figure 3-3 Spring minimum significant wave height.	19

Figure 3-4 Summer minimum significant wave height.	20

Figure 3-5 Fall minimum significant wave height.	20

Figure 3-6 Annual maximum significant wave height (HsAx).	21

Figure 3-7 Winter maximum significant wave height.	22

Figure 3-8 Spring maximum significant wave height.	22

Figure 3-9 Summer maximum significant wave height.	23

Figure 3-10 Fall maximum significant wave height.	23

Figure 3-11 Frequency of rough wave days (fHsRo).	24

Figure 3-12 Frequency of high wave days (fHsHi).	25

Figure 3-13 Frequency of top decile wave days (fHs90p).	25

Figure 3-14 Frequency of low decile wave days (fHs10p).	26

Figure 3-15 Top decile wave spell duration indicator (HHsDI).	26

Figure 3-16 Annual minimum wind speed.	27

Figure 3-17 Winter minimum wind speed.	27

Figure 3-18 Spring minimum wind speed.	28

Figure 3-19 Summer minimum wind speed.	28

Figure 3-20 Fall minimum wind speed.	29

Figure 3-21 Annual maximum wind speed (WsAx).	30

Figure 3-22 Winter maximum wind speed.	31

Figure 3-23 Spring maximum wind speed.	31

Figure 3-24 Summer maximum wind speed.	32

Figure 3-25 Fall maximum wind speed.	32

Figure 3-26 Frequency of calm wind days (fWsB0).	33

Figure 3-27 Frequency of near gale-force wind days (fWsB7).	34

Figure 3-28 Frequency of gale-force wind days (fWsB8).	34

Figure 3-29 Frequency of strong gale-force wind days (fWsB9).	35

Figure 3-30 Frequency of storm-force wind days (fWsB10).	35

Figure 3-31 Frequency of top decile wind days (fWs90p).	36

Figure 3-32 Frequency of low decile wind days (fWs10p).	36

Figure 3-33 Top decile wind spell duration indicator (HWsDI).	37

Figure 3-34 Low decile wind spell duration indicator (LWsDI).	37

Figure 3-35 January Mean Wind Direction	38

Figure 3-36 February Mean Wind Direction	38

Figure 3-37 March Mean Wind Direction	39

Figure 3-38 April Mean Wind Direction	39

Figure 3-39 May Mean Wind Direction	40

Figure 3-40 June Mean Wind Direction	40

Figure 3-41 July Mean Wind Direction	41

Figure 3-42 August Mean Wind Direction	41

Figure 3-43 September Mean Wind Direction	42

Figure 3-44 October Mean Wind Direction	42

Figure 3-45 November Mean Wind Direction	43

Figure 3-46 December Mean Wind Direction	43

Figure 4-1 Annual minimum significant wave height projected changes.	46

Figure 4-2 Winter minimum significant wave height projected changes.	47

Figure 4-3 Spring minimum significant wave height projected changes.	48

Figure 4-4 Summer minimum significant wave height projected changes.	49

Figure 4-5 Fall minimum significant wave height projected changes.	50

Figure 4-6 Annual maximum significant wave height (HsAx) projected changes.	52

Figure 4-7 Winter maximum significant wave height projected changes.	53

Figure 4-8 Spring maximum significant wave height projected changes.	54

Figure 4-9 Summer maximum significant wave height projected changes.	55

Figure 4-10 Fall maximum significant wave height projected changes.	56

Figure 4-11 Frequency of rough wave days (fHsRo) projected changes.	59

Figure 4-12 Frequency of high wave days (fHsHi) projected changes.	60

Figure 4-13 Frequency of top decile wave days (fHs90p) projected changes.	61

Figure 4-14 Frequency of low decile wave days (fHs10p) projected changes.	62

Figure 4-15 Top decile wave spell duration indicator (HHsDI) projected changes.	63

Figure 4-16 Annual minimum wind speed projected changes.	65

Figure 4-17 Winter minimum wind speed projected changes.	66

Figure 4-18 Spring minimum wind speed projected changes.	67

Figure 4-19 Summer minimum wind speed projected changes.	68

Figure 4-20 Fall minimum wind speed projected changes.	69

Figure 4-21 Annual maximum wind speed (WsAx) projected changes.	71

Figure 4-22 Winter maximum wind speed projected changes.	72

Figure 4-23 Spring maximum wind speed projected changes.	73

Figure 4-24 Summer maximum wind speed projected changes.	74

Figure 4-25 Fall maximum wind speed projected changes.	75

Figure 4-26 Frequency of calm wind days (fWsB0) projected changes.	78

Figure 4-27 Frequency of near gale-force wind days (fWsB7) projected changes.	79

Figure 4-28 Frequency of gale-force wind days (fWsB8) projected changes.	80

Figure 4-29 Frequency of strong gale-force wind days (fWsB9) projected changes.	81

Figure 4-30 Frequency of storm-force wind days (fWsB10) projected changes.	82

Figure 4-31 Frequency of top decile wind days (fWs90p) projected changes.	83

Figure 4-32 Frequency of low decile wind days (fWs10p) projected changes.	84

Figure 4-33 Top decile wind spell duration indicator (HWsDI) projected changes.	85

Figure 4-34 Low decile wind spell duration indicator (LWsDI) projected changes.	86

Figure 4-35 January mean wind direction projected changes.	87

Figure 4-36 February mean wind direction projected changes.	88

Figure 4-37 March mean wind direction projected changes.	88

Figure 4-38 April mean wind direction projected changes.	89

Figure 4-39 May mean wind direction projected changes.	89

Figure 4-40 June mean wind direction projected changes.	90

Figure 4-41 July mean wind direction projected changes.	90

Figure 4-42 August mean wind direction projected changes.	91

Figure 4-43 September mean wind direction projected changes.	91

Figure 4-44 October mean wind direction projected changes.	92

Figure 4-45 November mean wind direction projected changes.	92

Figure 4-46 December mean wind direction projected changes.	93

Figure 4-47 Root Mean Square Error of Simulated Historical Surface Wind speed - CMIP5 Models compared to CFSR Reanalysis.	94

Figure 5-1 Sea Surface Temperature 1981-2010 Climatology. DJF (top-left), MAM (top-right), JJA (bottom-left), SON (bottom-right).	103

Figure 5-2 Surface Air Temperature 1981-2010 Climatology. DJF (top-left), MAM (top-right), JJA (bottom-left), SON (bottom-right).	104

Figure 5-3 Frequency of Presence of Sea Ice, Week of March 19th (Canadian Ice Service, 2011)	105

Figure 5-4 Projections of relative sea-level rise by the year 2100 for the ensemble median (left) and 95th percentile (right) value of the high-emissions (RCP 8.5) scenario (from Lemmen et al., 2016 after James et al., 2014)	107

list of Appendices

APPENDIX A: 	HISTORICAL WAVE HEIGHT SUMMARY STATISTICS AND TRENDS

APPENDIX B: 	HISTORICAL EXTREME WAVE HEIGHT INDICES AND TRENDS

APPENDIX C: 	HISTORICAL WIND SPEED SUMMARY STATISTICS AND TRENDS

APPENDIX D: 	HISTORICAL EXTREME WIND SPEED INDICES AND TRENDS

APPENDIX E: 	HISTORICAL MONTHLY WIND DIRECTION

APPENDIX F: 	RCP 4.5 PROJECTED CHANGES IN WAVE HEIGHT SUMMARY STATISTICS

APPENDIX G: 	RCP 4.5 PROJECTED CHANGES IN EXTREME WAVE HEIGHT INDICES

APPENDIX H: 	RCP 4.5 PROJECTED CHANGES IN WIND SPEED SUMMARY STATISTICS

APPENDIX I: 	RCP 4.5 PROJECTED CHANGES IN EXTREME WIND SPEED INDICES

APPENDIX J: 	RCP 4.5 PROJECTED CHANGES IN MONTHLY WIND DIRECTION

APPENDIX K: 	RCP 8.5 PROJECTED CHANGES IN WAVE HEIGHT SUMMARY STATISTICS

APPENDIX L: 	RCP 8.5 PROJECTED CHANGES IN EXTREME WAVE HEIGHT INDICES

APPENDIX M: 	RCP 8.5 PROJECTED CHANGES IN WIND SPEED SUMMARY STATISTICS

APPENDIX N: 	RCP 8.5 PROJECTED CHANGES IN EXTREME WIND SPEED INDICES

APPENDIX O: 	RCP 8.5 PROJECTED CHANGES IN MONTHLY WIND DIRECTION

# Introduction

This study focuses on the recent climatology and projected effects of climate change on wind and waves in Area Response Plan (ARP) Pilot Area 4: Port Hawkesbury and the Strait of Canso. This report is one of four produced during this study, each of which focuses on a different ARP Pilot Area in Canadian coastal waters (Figure 1-1), all of which were selected due to their high volume of marine traffic. Other areas include the southern portion of British Columbia, the St. Lawrence River (Montreal to Anticosti Island), as well as Saint John and the Bay of Fundy. This project supports the Government of Canada’s ARP Initiative which aims to strengthen national marine oil spill preparedness and response.

Figure 1-1 Area Response Plans Pilot Areas (ECCC)

The purpose of this report, and the study on which it is based, is to provide “operational science to inform decision making for effective emergency preparedness and response.” Ultimately the role of the Historical and future projected wind speed and direction information provided here is to reduce the environmental impact of a marine emergency in the area of focus. The analysis presented here may be used to inform the development and use of emergency response plans and spill trajectory and dispersion models, guide risk mitigation efforts for marine wildlife and migratory birds, and more generally provide insight into marine operating conditions.

Climatological analysis components included the calculation of various descriptive statistics for significant wave heights and wind speeds as well as numerous extreme value indices, defined by Environment and Climate Change Canada (ECCC). Spatial statistics and indices were determined for each month, season and year. Long-term means and linear trends were then determined for all calculated values. Results of all means and trends were then mapped. Analysis and maps were also produced for changes in mean wind direction for each month.

Climate change analysis was conducted using an ensemble of latest generation Coupled Model Intercomparison Project (CMIP5) global climate model (GCM) output. The climate change signal for all statistics, indices and wind directions mentioned above were calculated for two greenhouse gas emissions scenarios (RCP 4.5 and RCP 8.5) and two planning horizons (mid and end 21st century). Minimum, mean and maximum ensemble projection results were then mapped to ensure a broad range of climate projection uncertainty was represented.

## Report Organization

The report begins with a description of the wind and wave climatological analysis in Section 2.0, which includes an overview of all the datasets used. Section 3.0 presents the results from the historical long-term mean and trend analysis for all statistics and indices of interest, while the associated climate change analysis results are found in Section 4.0. The impact of the historical and projected climates on the marine transportation industry and oil spill response is found in Section 5.0. A summary is provided in Section 6.0.

The appendices are organized parallel to the corresponding report sections and each begins with a table of contents. For those covering summary statistics (Appendices A, C, F, H, K, and M) each long-term mean is first presented for all 17 timeframes and is followed by the respective linear trends. The maps being with the minimum statistic, progress upwards through the percentiles, and finish with the maximum. Each statistic has a common legend across all 17 timeframes. Extreme wave and wind indices (in Appendices B, D, G, I, L, and N) are presented in the order listed in Tables 2-1 and 2-2, respectively. Monthly wind direction means and changes (Appendices E, J, and O) are presented in chronological order.

# Wind and Wave Climatological Analysis

The climatological and climate change analyses are based on a series of statistics and indices. Historical analysis uses the MSC50 data to determine long-term mean values and trends of the seven statistics as well as the nine extreme wave height and 16 extreme wind speed indices. The climate change analysis uses CMIP5 climate models to investigate the range of potential climate change impacts on the statistics and indices.

A selection of digital maps from the historical analysis are available in Section 3.0, while maps showing climate projection results are presented in Section 4.0. The full collection of maps are contained within the appendices.

## Summary Statistics

The seven statistics of interest include minimum, maximum, 5th, 25th, 50th, 75th, and 95th percentiles. These statistics are to be calculated over seventeen timeframes include 12 monthly, four seasonal and an annual timeframes. The long-term means and linear trends for all statistics and timeframes were calculated and plotted.

The climate change signal for each statistic and timeframe was also calculated. The climate change analysis focuses on three climate model time-slices: a base period (1986-2005); a mid-century future (2046-2065); and an end of century future (2081-2100). Differences between the long-term mean of the future periods and the base period (all statistics and time frames) are investigated to determine the impacts of climate change.

## Indices

Extreme wave height and wind speed indices are defined in Tables 2-1 and 2-2 respectively. Each set is based on the daily maximum significant wave height and surface wind speed respectively. Aside from monthly and annual maxima (also covered by the summary statistics above) the indices focus on the frequency of certain thresholds or deciles being exceeded. The long-term means and linear trends for all indices were calculated and plotted. In a manner similar to the summary statistics, the climate change signal for each index was also calculated.

Table 2-1: Extreme Wave Height Indices

| ID | Indicator name | Definitions | Unit |
| --- | --- | --- | --- |
| HsMx | Monthly max Hs | Monthly maximum value of significant wave height (Hs) | m |
| HsAx | Annual max Hs | Annual maximum value of significant wave height (Hs) | m |
| HsRo | Rough wave days | Annual count of days when daily max Hs > 2.5 m | days |
| HsHi | High wave days | Annual count of days when daily max Hs > 6 m | days |
| fHsRo | Frequency of rough wave days | Annual percentage of days when daily max Hs > 2.5 m | % |
| fHsHi | Frequency of high wave days | Annual percentage of days when daily max Hs > 6 m | % |
| fHs90p | Frequency of top decile wave days | Annual percentage of days when daily max Hs > 90th percentile of base period (1986–2005) (all calendar days share the same percentiles) | % |
| fHs10p | Frequency of low decile wave days | Annual percentage of days when daily max Hs < 10th percentile of base period (1986–2005) (all calendar days share the same percentiles) | % |
| HHsDI | Top decile wave spell duration indicator | Annual count of days with at least 2 consecutive days when daily max Hs > 90th percentile of base period (1986–2005) (all calendar days share the same percentiles) | days |

Table 2-2: Extreme Wind Indices

| ID | Indicator name | Definitions | Unit |
| --- | --- | --- | --- |
| WsMx | Monthly max wind speed | Monthly maximum value of wind speed (Ws) | m/s |
| WsAx | Annual max wind speed | Annual maximum value of wind speed (Ws) | m/s |
| WsB0 | Calm wind days | Annual count of days when daily max Ws < 0.514 m/s (1 Knot, Beaufort Scale 0) | days |
| WsB7 | Near gale-force wind days | Annual count of days when daily max Ws > 14.403 m/s (Beaufort Scale 7) | days |
| WsB8 | Gale-force wind days | Annual count of days when daily max Ws > 17.222 m/s (Beaufort Scale 8) | days |
| WsB9 | Strong gale-force wind days | Annual count of days when daily max Ws > 20.833 m/s (Beaufort Scale 9) | days |
| WsB10 | Storm-force wind days | Annual count of days when daily max Ws > 24.722 m/s (Beaufort Scale 10) | days |
| fWsB0 | Frequency of calm wind days | Annual percentage of days when daily max Ws < 0.514 m/s (1 Knot, Beaufort Scale 0) | % |
| fWsB7 | Frequency of near gale-force wind days | Annual percentage of days when daily max Ws > 14.403 m/s (Beaufort Scale 7) | % |
| fWsB8 | Frequency of gale-force wind days | Annual percentage of days when daily max Ws > 17.222 m/s (Beaufort Scale 8) | % |
| fWsB9 | Frequency of strong gale-force wind days | Annual percentage of days when daily max Ws > 20.833 m/s (Beaufort Scale 9) | % |
| fWsB10 | Frequency of storm-force wind days | Annual percentage of days when daily max Ws > 24.722 m/s (Beaufort Scale 10) | % |
| fWs90p | Frequency of top decile wind days | Annual percentage of days when daily max Ws > 90th percentile of base period (1986–2005) (all calendar days share the same percentiles) | % |
| fWs10p | Frequency of low decile wind days | Annual percentage of days when daily max Ws < 10th percentile of base period (1986–2005) (all calendar days share the same percentiles) | % |
| HWsDI | Top decile wind spell duration indicator | Annual count of days with at least 2 consecutive days when daily max Ws > 90th percentile of base period (1986–2005) (all calendar days share the same percentiles) | days |
| LWsDI | Low decile wind spell duration indicator | Annual count of days with at least 2 consecutive days when daily max Ws < 10th percentile of base period (1986–2005) (all calendar days share the same percentiles) | days |

## Description of Datasets and Models

All datasets used in the analysis are described below.

### MSC50 Atlantic

The MSC50 (MSC06min subset), wind and wave hindcast dataset (Swail et al. 2006) represents the state of the art and most comprehensive wind and wave database for Eastern Canada waters.  The MSC50 M6 Eastern Canada Subset domain covers from approximately 74°W to 42°W and from 40°N to 52°N at 0.1° resolution (approximately 10 km). The dataset covers hourly wind and wave parameters, from 1954 to 2015, for the North Atlantic Ocean and includes consideration of period with sea ice coverage (Swail et al. 2006).

Ice concentration data considered is a mean monthly value through 1961 inclusive (Walsh and Johnson, 1979 as reported in Swail et al., 2006) and then Canadian Ice Service (CIS) mean weekly ice concentration for 1962 onwards. That is, ice is applied for an entire calendar week, e.g., one would see something like the 1st to 7th of the month is ice, then 8th to 14th isn’t, etc.

While the MSC50 hindcast includes the below parameters, wind speed and direction as well as significant wave height were the only ones analyzed in this report.

Wind direction, WD

Wind speed, WS, 1-hour average

For each of the primary partition (sea), secondary partition (swell), and total (sea+swell) wave spectrum:

variance of spectrum (M0, spectral moment of order 0)

peak spectral period, Tp

vector mean direction of wave spectrum, to which waves are travelling, VMD

Significant wave height, (equal to four times the square root of M0), Hs

Spectral moment of order 2 ( M2)

Dominant direction, DD

Angular Spreading Function

In-Line Variance Ratio, or directional spreading

For a complete data inventory see http://www.meds-sdmm.dfo-mpo.gc.ca/isdm-gdsi/waves-vagues/MSC50-eng.html.

### NCEP CFSR

The National Centers for Environmental Prediction (NCEP) Climate Forecast System Reanalysis (CFSR) dataset is the latest generation reanalysis dataset, developed with the intention of superseding the NCEP-NCAR Reanalysis in both scope and quality (Saha et al 2010). It is a global, high resolution (~38 km for atmospheric variables) coupled atmosphere-ocean-land-surface-sea ice system developed to provide an improved estimate of the state of these domains over the period 1979-2015. While the only variables used from for this study relate to surface winds, CFSR also publishes hourly, 0.5° latitude x 0.5° longitude output products for a variety of atmospheric, oceanic, and land surface variables. CFSR wind data is used in this study as a basis of evaluation for CMIP5 (below) simulated historical wind data.

Additional information can be found at http://cfs.ncep.noaa.gov/cfsr/.

### CMIP5 Global Climate Models

The Coupled Model Intercomparison Project Phase 5 (CMIP5) is a coordinated global climate modeling effort involving 20 modeling groups from around the world. The output from the CMIP5 models provides the basis of the Intergovernmental Panel on Climate Change (IPCC) Fifth Assessment Report (AR5), (IPCC, 2014). CMIP5 promotes a standardized and intercomparable approach to climate modeling in order to evaluate the effectiveness of simulating past climates, encompass a broad range of model driven climate change projection uncertainty, as well as facilitating research into factors responsible for differences in model projections and representation of climate processes.

Four CMIP5 global climate models (GCMs) were selected for inclusion in this study. Pertinent details are listed in Table 2-3 below. Ensemble Runs distinguish between closely related simulations from a single model, where “r#” is the ‘realization number’ and represents ensemble members that were initialized by differing initial conditions (providing a representation of internal variability of the climate model). See Section 2.3 in http://cmip-pcmdi.llnl.gov/cmip5/docs/cmip5_data_reference_syntax.pdf for details pertaining to i# and p#, the ‘initialization method indicator’ and ‘perturbed physics’ number, respectively.

Table 2-3: CMIP5 Model Information

| Model | Modeling Institution | Country | Reporting Frequency | Horizontal Resolution* (Latitude Longitude) | Ensemble Runs Included |
| --- | --- | --- | --- | --- | --- |
| GFDL-ESM2M | National Oceanic and Atmospheric Administration (NOAA) Geophysical Fluid Dynamics Laboratory (GFDL) | United States | Waves: 6 hrs Wind: 3 hrs | 2.0225° 2.5° | r1i1p1 |
| IPSL-CM5A-MR | Institut Pierre Simon Laplace (IPSL) | France | Waves: 6 hrs Wind: 3 hrs | 1.2676° 2.5° | r1i1p1 |
| MIROC5 | Atmosphere and Ocean Research Institute (AORI), The University of Tokyo | Japan | Waves: 6 hrs Wind: 3 hrs | 1.4008° 1.40625° | r1i1p1 r2i1p1 r3i1p1 |
| MRI-CGCM3 | Meteorological Research Institute (MRI) | Japan | Waves: 6 hrs Wind: 3 hrs | 1.12148° 1.125° | r1i1p1 |

*Calibrated to the equatorial region, higher latitude variations may occur.

# Historical Wind and Waves Climate and Trend Analysis

This section discusses historical means and trends of significant wave height and surface wind speeds, as represented by the 1954-2015 coverage of the MSC50 hindcast dataset.

The following subsections present maps of the analysis results for the seven summary statistics across all 17 timeframes as well as the extreme wind and wave indices (defined in Tables 2-1 and 2-2). These include the calculation of long-term means as well as linear trends (using Kendall’s estimator, as described in Wang and Swail, 2001). Subsections are organized according to minimum, maximum and index values for significant wave heights and wind speeds. Means and changes in mean monthly wind direction are also presented.

Each figure consists of a map of the long term-mean (1954-2015) of the given statistic or index for the given timeframe (left side) as well as a map the linear trend over the same timeframe (right side). The ‘points of significance’ refer to those grid cells that experienced a statistically significant linear trend at the 5% level (i.e., there is less than a 5% chance that the trend is due to random inter-annual noise). Each grid cell in maps of this section correspond to a single MSC50 MSC06min node (see Section 2.3.1), which have a resolution of 0.1° latitude and longitude.

Table 3-1: Historical Significant Wave Height Statistics Summary

| Significant Wave Height Statistics | Minimum Wave Height |  | Median Wave Height |  | Maximum Wave Height |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Long-term mean (m) | Significant Trend (m/decade) | Long-term mean (m) | Significant Trend (m/decade) | Long-term mean (m) | Significant Trend (m/decade) |
| Annual | 0.0 to 0.4 | 0.02 to 0.03* | 0.5 to 1.75 | 0.03 to 0.07 | 3.0 to 9.0 | 0.2 to 0.3 |
| Winter | 0.05 to 0.5 | 0.01 to 0.07 | 0.7 to 2.5 | 0.02 to 0.12* | 3.0 to 8.3 | 0.15 to 0.25* |
| Spring | 0.0 to 0.4 | 0.01 to 0.04* | 0.4 to 1.6 | 0.02* | 2.0 to 7.0 | - |
| Summer | 0.05 to 0.5 | 0.005 to 0.01* | 0.25 to 1.3 | 0.02 | 1.5 to 4.75 | - |
| Fall | 0.05 to 0.6 | 0.01 to 0.02* | 0.5 to 2.0 | 0.02 to 0.04 | 2.0 to 7.0 | - |

* Significant trend only in portions of the region, see maps for more details

Table 3-2: Historical Surface Wind Speed Statistics Summary

| Surface Wind Speed Statistics | Minimum Wind Speed |  | Median Wind Speed |  | Maximum Wind Speed |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Long-term mean (m/s) | Significant Trend (m/s/decade) | Long-term mean (m/s) | Significant Trend (m/s/decade) | Long-term mean (m/s) | Significant Trend (m/s/decade) |
| Annual | 0.2 to 0.5 | 0.01 to 0.02* | 7 to 8 | -0.01 to 0.03 | 22 to 25 | -0.03 to 0.1* |
| Winter | 0.7 to 1.4 | - | 9.3 to 10.2 | - | 22 to 23.5 | - |
| Spring | 0.2 to 0.6 | 0.01 to 0.03* | 6.6 to 7.5 | -0.09 to -0.07* | 20 to 22 | - |
| Summer | 0.15 to 0.4 | 0.01 to 0.03* | 5.0 to 5.7 | - | 15 to 16 | - |
| Fall | 0.5 to 1.1 | 0.05 to 0.08 | 7.5 to 7.9 | 0.11 | 20 to 21.5 | 0.28 to 0.33* |

* Significant trend only in portions of the region, see maps for more details

Table 3-1 and 3-2 present representative values from the figures in this section and the Appendices, for annual and seasonal long-term means and trends. Summary Tables of extreme wave height indices (Table 3-3) and extreme wind speed indices (Table 3-4), are found in Section 3.3 and 3.6, respectively. While maps include a small region of the Northumberland Strait and St. George’s Bay, northwest of Nova Scotia, all discussion and summary table values focus on the region southeast of Nova Scotia, as outlined in Figure 1-1.

A detailed inspection of the results will reveal that annual minimum maps have lower values than the corresponding seasonal maps, which have lower values than the respective monthly maps. The primary reason for the difference is that the minimum wave height (or wind speed) value for any given season does not necessarily occur in the same month every year. As such the seasonal long-term mean maps will necessarily be lower than each of the three months that contribute. Similarly, the annual long-term means will necessarily be lower than each of the four seasons. The opposite is true for the annual, seasonal and monthly maximum maps. The annual values will necessarily be higher than the seasonal, which will in turn be higher than the corresponding monthly maps.

## Minimum Wave Heights

The minimum annual significant wave heights (Figure 3-1) range from 0 m near the Strait of Canso to 0.15 m along much of the coastal area up to 0.4 m further offshore. There is a significant positive trend of 0.02 to 0.03 m per decade in the western half of the region.

Figure 3-1 Annual minimum significant wave height.

There are some differences between seasons, as seen in Figures 3-2 to 3-5. The minimum wave heights are largest in summer and fall (around 0.5 to 0.6 m) though offshore regions in winter have similar values. The lowest values are near the Strait of Canso and remain consistently below 0.1 m for the entire year. Positive significant trends were found for all seasons, though not throughout the entire region.

Figure 3-2 Winter minimum significant wave height.

Figure 3-3 Spring minimum significant wave height.

Figure 3-4 Summer minimum significant wave height.

Figure 3-5 Fall minimum significant wave height.

All maps for minimum and maximum, significant wave heights, as well as those for 5th, 25th, 50th, 75th, and 95th percentile wave heights for all seasons, months, and annual timeframes can be found in Appendix A.

## Maximum Wave Heights

The annual maximum value of Hs (HsAx) is close to 3 m approaching the Strait of Canso, in the range of 5-7 m along most of the southeast coast, and nearly 9 m in regions furthest offshore. This is trending upwards in most offshore regions at a rate of 0.2-0.3 m per decade. Most positive trends near the coast were not statistically significant. See Figure 3-6 for more details.

Figure 3-6 Annual maximum significant wave height (HsAx).

Monthly maximum values of Hs (HsMx) experience a clear annual cycle (figures available in Appendix A). Values are highest in regions furthest offshore with December and January seeing values on the order of 7.5 m and July having values 3.5 m and below. In sheltered areas close to the Strait of Canso, each month has relatively consistent HsMx in the 1-3 m range with July on the lower end of the spectrum and January on the higher end. February is the only month with significant trends throughout the entire area, ranging from 0.3-0.6 m per decade near the coast (highest values in the southwest) and 0.2 m per decade further offshore. April, June and October have isolated small patches of significant trends, each with values on the order of 0.1 m per decade.

Figures 3-7 through 3-10 present long-term mean and trends for winter, spring, summer and fall respectively.

Figure 3-7 Winter maximum significant wave height.

Figure 3-8 Spring maximum significant wave height.

Figure 3-9 Summer maximum significant wave height.

Figure 3-10 Fall maximum significant wave height.

All maps for minimum and maximum, significant wave heights, as well as those for 5th, 25th, 50th, 75th, and 95th percentile wave heights for all seasons, months, and annual timeframes can be found in Appendix A.

## Extreme Wave Height Indices

Maps of long-term means and linear trends of extreme wave height indices calculated from daily maximum wave heights (defined in Table 2-1) are presented in this section. Table 3-3 provides a summary with representative values corresponding to each plot below.

Table 3-3: Historical Extreme Wave Height Indices Summary

| Extreme Wave Height Indices* | fHsRo | fHsHi | fHs90p | fHs10p | HHsDI |
| --- | --- | --- | --- | --- | --- |
| Long-term mean | 2 to 45 % | 0 to 3 % | 9 to 10 % | 10 to 16 % | 26 to 29 days |
| Significant Trend | 1 to 1.5** % pts / decade | 0.125** % pts / decade | 0.4 to 0.7** % pts / decade | -3.0 to -1.0 % pts / decade | 1.2 to 2.0** days / decade |

* fHsRo - Frequency of rough wave days (Hs > 2.5 m)
  fHsHi - Frequency of high wave days (Hs > 6 m)
  fHs90p - Frequency of top decile wave days
  fHs10p - Frequency of low decile wave days
  HHsDI - Top decile wave spell duration indicator
** Significant trend only in portions of the region, see maps for more details

The frequency of rough wave days (fHsRo, Figure 3-11), when the daily maximum Hs is greater than 2.5 m, ranges from less than 2% near the Strait of Canso to 15% along the southeast coast and up to 45% and greater further offshore. This has been trending upwards at a rate of roughly 1 percentage points per decade offshore Cape Breton and closer to 1.5% per decade in places offshore mainland Nova Scotia.

As a point of reference, 10% corresponds to just over 36 days, 15% nearly 55 days and 50% over 182 days. One day is roughly 0.27% of the year while one week is around 1.9%.

Figure 3-11 Frequency of rough wave days (fHsRo).

Figure 3-12 Frequency of high wave days (fHsHi).

The frequency of high wave days (fHsHi, Figure 3-12), when the daily maximum Hs is greater than 6 m,  ranges from 0% near the Strait of Canso and along other parts of the coast, to over 3% further offshore. This is trending upwards in offshore regions at a rate of roughly 0.125 percentage points per decade.

The annual percentage of days when daily max Hs > 90th percentile of the base period (1986–2005) Hs (fHs90p, Figure 3-13), is around 9% near the coastline and approaches 10% in the southern offshore regions. This is trending upwards by roughly 0.5-0.7% per decade near the Strait of Canso and in places near the southwest coastline. Offshore trends are around 0.4 percentage points per decade

Figure 3-13 Frequency of top decile wave days (fHs90p).

Figure 3-14 Frequency of low decile wave days (fHs10p).

Annual percentage of days when daily max Hs < 10th percentile of base period Hs (fHs10p, Figure 3-14) ranges from 13-16% along most of the coastline down to 10-12% in southern offshore regions. This is trending downward by -3 to -1 percentage points per decade through ARP Area 4.

Annual count of days with at least 2 consecutive days when daily max Hs > 90th percentile of base period Hs (HHsDI, Figure 3-15), is between 27-28 days near most of the southeast coast, except in more sheltered regions where the count is closer to 26 days. Southern offshore regions see counts higher than 29 days. This is trending upward in regions near the Strait of Canso at 1.5-2 days per decade and at a rate of 1.2 in eastern offshore regions.

Figure 3-15 Top decile wave spell duration indicator (HHsDI).

All maps of extreme wave height indices for the historical analysis can also be found in Appendix B.

## Minimum Wind Speeds

Maps of long-term means and linear trends for minimum wind speeds are presented in this section. See Table 3-2 for a summary of values from the maps below.

Figure 3-16 Annual minimum wind speed.

The minimum annual wind speed ranges from 0.2-0.5 m/s across the region, with a slightly positive significant trend of 0.01-0.02 m/s per decade, as shown in Figure 3-16.There is a noticeable annual cycle in minimum wind speeds, as can be seen in Figures 3-17 to 3-20. The lowest values are in summer (0.15-0.4 m/s), which increase into the fall (0.5-1.1 m/s), peak in the winter (0.7-1.4 m/s) and drop back again in the spring (0.2-0.6 m/s). Small positive significant trends were found for every season, with the exception of winter.

Figure 3-17 Winter minimum wind speed.

Figure 3-18 Spring minimum wind speed.

Figure 3-19 Summer minimum wind speed.

Figure 3-20 Fall minimum wind speed.

All maps for minimum and maximum wind speeds, as well as those for 5th, 25th, 50th, 75th, and 95th percentile wind speeds for all seasons, months, and annual timeframes can be found in Appendix C.

## Maximum Wind Speeds

Maps of long-term means and linear trends for maximum wind speeds are presented in this section. See Table 3-2 for a summary of values from the maps below.

The annual maximum wind speed (WsAx, Figure 3-21) ranges from slightly greater than 22 m/s near the Strait of Canso up to nearly 25 m/s in eastern offshore regions. This is significantly trending slightly upwards at a rate less than 0.1 m/s per decade in eastern regions and slightly downwards approaching -0.03 m/s per decade in isolated southern regions.

Figure 3-21 Annual maximum wind speed (WsAx).

Monthly maximum values of Ws (WsMx, Figures available in Appendix C) experience a clearly defined annual cycle. Highest values occur in offshore regions of January and December (22 m/s), while July sees only 13 m/s offshore. In western and sheltered areas July’s values are as low as 12 m/s while December and January experience approximately 20 m/s. Again, February is the only month showing statistically significant trends throughout the region with values around 0.075 m/s per decade in the west up to 0.2 m/s further east. June, September and October have small isolated significant trends typically with values < 0.1 m/s per decade (and September showing slightly negative trends).

Figures 3-22 through 3-25 present long-term mean and trends for winter, spring, summer and fall respectively.

Figure 3-22 Winter maximum wind speed.

Figure 3-23 Spring maximum wind speed.

Figure 3-24 Summer maximum wind speed.

Figure 3-25 Fall maximum wind speed.

All maps for minimum and maximum wind speeds, as well as those for 5th, 25th, 50th, 75th, and 95th percentile wind speeds for all seasons, months, and annual timeframes can be found in Appendix C.

## Extreme Wind Speed Indices

Maps of long-term means and linear trends of extreme wind speed indices calculated from daily maximum wind speeds (defined in Table 2-2) are presented in this section. Table 3-4 provides a summary with representative values corresponding to each plot below.

Table 3-4: Historical Extreme Wind Speed Indices Summary

| Extreme Wind Speed Indices* | fWsB0 (%) | fWsB7 (%) | fWsB8 (%) | fWsB9 (%) | fWsB10 (%) | fWs90p (%) | fWs10p (%) | HWsDI (days) | LWsDI (days) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Long-term mean | 0 | 14. 5 to 21.5 | 4.75 to 8.75 | 0.6 to 1.9 | 0.025 to 0.15 | 9.9 to 10.0 | 9.95 to 10.0 | 25 to 26 | 22 to 22.5 |
| Significant Trend | - | - | - | - | - | - | - | - | - |

* fWsB0 - Frequency of calm wind days (Ws < 0.5 m/s)
fWsB7 - Frequency of near gale-force wind days (Ws > 14.4 m/s)
fWsB8 - Frequency of gale-force wind days (Ws > 17.2 m/s)
fWsB9 - Frequency of strong gale-force wind days (Ws > 20.8 m/s)
fWsB10 - Frequency of storm-force wind days (Ws > 24.7 m/s)
fWs90p - Frequency of top decile wind days 
fWs10p - Frequency of low decile wind days 
HWsDI - Top decile wind spell duration indicator 
LWsDI - Low decile wind spell duration indicator

The annual percentage of days when daily max Ws < 0.514 m/s (1 knot, Beaufort Scale 0) (fWsB0, Figure 3-26) is 0% and there is no discernable trend.

Figure 3-26 Frequency of calm wind days (fWsB0).

As a point of reference, 10% corresponds to just over 36 days, 15% nearly 55 days and 50% over 182 days. One day is roughly 0.27% of the year while one week is around 1.9%. For further reference 1 m/s equates to 3.6 km/hr.

The annual percentage of days when daily max Ws > 14.403 m/s (Beaufort Scale 7) (fWsB7, Figure 3-27) is approximately 16.5% near the Strait of Canso and ranges from 14.5-19.5% from west to east along the coastal regions and up to 21.5% in eastern offshore areas. While trends in the west are close to zero, and trends east of the Strait of Canso are in the range of 0.25 percentage points per decade no trends were found to be statistically significant.

Figure 3-27 Frequency of near gale-force wind days (fWsB7).

The pattern of annual percentage of days when daily max Ws > 17.222 m/s (Beaufort Scale 8) (fWsB8, Figure 3-28) and trends follow similar geographical distributions as fWsB7, with lower percentages in the west (4.75%) and higher percentages in the east (8.75%). No statistically significant trends were found.

Figure 3-28 Frequency of gale-force wind days (fWsB8).

Similar to fWsB7 and fWsB8, the annual percentage of days when daily max Ws > 20.833 m/s (Beaufort Scale 9) (fWsB9, Figure 3-29) increases from east (0.6%) to west (1.9%), with no statistically significant trend.

Figure 3-29 Frequency of strong gale-force wind days (fWsB9).

Continuing the pattern, the annual percentage of days when daily max Ws > 24.722 m/s (Beaufort Scale 10) (fWsB10, Figure 3-30) increases from 0.025% in the east to 0.15% in the west, with values close to zero in some sheltered areas. No trend was found.

Figure 3-30 Frequency of storm-force wind days (fWsB10).

The annual percentage of days when daily max Ws > 90th percentile of base period Ws (1986–2005)  (fWs90p, Figure 3-31) is in the small range of 9.9-10% for the entire area. Trends were found to be close to zero with no statistical significance.

Figure 3-31 Frequency of top decile wind days (fWs90p).

The annual percentage of days when daily max Ws < 10th percentile of base period Ws (fWs10p, Figure 3-32) also has a small range of values, predominantly from 9.95-10%. Small but non-significant trends were found.

Figure 3-32 Frequency of low decile wind days (fWs10p).

The annual count of days with at least 2 consecutive days when daily max Ws > 90th percentile of base period Ws (HWsDI, Figure 3-33) is predominantly 25-26 days. Small non-significant trends were found.

Figure 3-33 Top decile wind spell duration indicator (HWsDI).

The annual count of days with at least 2 consecutive days when daily max Ws < 10th percentile of base period Ws (LWsDI, Figure 3-34) is 22-22.5 days for most areas with slightly higher and lower pockets in the west and east respectively.

Figure 3-34 Low decile wind spell duration indicator (LWsDI).

All maps of extreme wind speed indices for the historical analysis can also be found in Appendix D.

## Monthly Wind Direction

ARP Area 4 resides in a predominantly westerly wind regime, meaning the average wind direction blows from the west and towards the east. Figures 3-35 to 3-46 below show the monthly long-term (1954-2015) mean wind directions (left maps) together with the change in mean wind direction from 1980-1995 to 2000-2015 (right maps). Grid cells with a statistical significance of change (i.e., there was a less than 5% chance the change was random noise) are noted.

Figure 3-35 January Mean Wind Direction

Figure 3-36 February Mean Wind Direction

The annual cycle of mean wind direction is east-south-eastward in January through March (Figures 3-35 to 3-37, respectively), to more eastward in April (Figure 3-38), to north-eastward in May through August (Figures 3-39 to 3-42, respectively), east-north-eastward in September (Figure 3-43), back to eastward in October and November (Figures 3-44 and 3-45, respectively), and east-south-eastward again in December (Figure 3-46).

Most months experienced some significant change in mean wind direction from 1980-1995 to 2000-2015, though many were relatively small shift (between 14° clockwise and anti-clockwise). April was unique in that is showed significant changes in both directions, with clockwise changes closer to shore (up to 21°) and anti-clockwise changes further offshore (up to 14°). May exhibited the largest changes with clockwise rotation of up to 35° in offshore areas.

Figure 3-37 March Mean Wind Direction

Figure 3-38 April Mean Wind Direction

Figure 3-39 May Mean Wind Direction

Figure 3-40 June Mean Wind Direction

Figure 3-41 July Mean Wind Direction

Figure 3-42 August Mean Wind Direction

Figure 3-43 September Mean Wind Direction

Figure 3-44 October Mean Wind Direction

Figure 3-45 November Mean Wind Direction

Figure 3-46 December Mean Wind Direction

All historical wind direction maps are also available in Appendix E.

# Projected Future Changes based on four CMIP5 Models

This section discusses the investigation into the climatic changes between a base period of 1986-2005 and two future periods: mid-century, represented by the years 2046-2065, and; end-century, represented by 2081-2100. The climate change signal is determined by the difference between a climate model’s future projection and its base period simulation. The analysis included both a moderate an aggressive greenhouse gas emissions scenario, RCP 4.5 and RCP 8.5, respectively. While all results and maps for each scenario can be found in the appendices, the discussion in this section will focus exclusively on the RCP 8.5, which represents the higher GHG pathway and will therefore contain the stronger climate change signal.

An ensemble of climate models is used in the analysis in order to obtain a representation of climate projection uncertainty. This uncertainty is represented by the spread in the results of the climate change analysis across all individual models. Four global climate models (GCMs) are employed, details of which are provided in Table 2-3. Three different ensemble runs from the MIROC5 model are also incorporated (r1i1p1, r2i1p1, and r3i1p1) providing a total of six ensemble members.

It needs to be stressed that only four coarse resolution CMIP5 GCMs were used in the study. The 1° grid resolution for waves resulted in poor wave data coverage in areas directly adjacent to coastlines. This relatively small ensemble size and coarse resolution leads to large uncertainties in the projected results. All accompanying discussion herein refers to only those grid-points shown in the maps and results should be interpreted through this lens.

Each figure in this section consists of six maps: three rows and two columns. The middle row presents the ensemble mean projection, while the top and bottom rows show the ensemble minimum and maximum projections, respectively. The left column contains the impacts of climate change out to the mid-21st century (difference between 2046-2065 and 1986-2005), while the right column displays the impacts out to the end-21st century (difference between 2081-2100 and 1986-2005). Maps from the top-left to bottom-right will show the full ensemble spread of analysis results for a given change in waves or wind for the RCP 8.5 scenario.

Following Section 3, the climate change analysis included the calculation of seven statistics of surface wind speeds and significant wave heights for 17 timeframes as well as extreme wind and wave indices for the three time-slices. The mean wind direction for each month for the three time-slices was also calculated. As mentioned above, differences between future time-slices and the base period were calculated for each model to give an ensemble of projected climatological changes of extreme wind and wave indices.

The following subsections present maps of the analysis results across annual and seasonal timeframes. Subsections are organized according to minimum, maximum and index values for significant wave heights and wind speeds. Changes in mean monthly wind direction are also covered. While each map covers a large region of Canada’s Atlantic coastal waters, all discussion and summary tables focus on the ARP Pilot Area 4, just southeast of Cape Breton and eastern mainland Nova Scotia. The larger region is provided in the maps to provide context for any changes occurring in Area 4.

Section 4.8 shows the evaluation of simulated wind from the climate models against NCEP CFSR reanalysis data.

## Minimum Wave Heights

This section presents the ensemble projected changes in minimum wave heights for the RCP 8.5 GHG scenario for the mid-century (2046-2061) and end-century (2081-2100). As described in Section 4.0, Figures 4-1 to 4-5 each contain six maps representing ensemble minimum (top row), mean (middle row) and maximum (bottom row) projections.

A summary of values representative of ARP Pilot Area 4 from the following figures are shown in Table 4-1. Overall, a small decrease or little change in minimum significant wave height is projected.

Table 4-1: Projected Changes in Minimum Wave Height Summary

| Climate Change Signal (m) | Ensemble Minimum |  | Ensemble Mean |  | Ensemble Maximum |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Mid-Century | End-Century | Mid-Century | End-Century | Mid-Century | End-Century |
| Annual | -0.06 to 0 | -0.08 to 0 | -0.03 to 0 | -0.06 to 0 | 0 | -0.03 to 0 |
| Winter | -0.08 to 0 | -0.08 to 0 | -0.03 to 0 | -0.06 to 0 | 0 to 0.03 | 0 |
| Spring | -0.06 to 0 | -0.06 to 0 | -0.03 to 0 | 0 | 0 to 0.06 | 0 to 0.06 |
| Summer | -0.06 to 0 | -0.06 to 0 | 0 | 0 | 0 to 0.03 | 0 to 0.03 |
| Fall | -0.06 | -0.09 to -0.06 | -0.03 | -0.06 | 0 | -0.03 |

While there are some differences between the four seasons, they typically are not large and the annual climate change signal can be considered representative for all seasons. The ensemble minimum projected changes in minimum annual significant wave height for mid-century are between -0.06 and 0 m, and the ensemble mean shows a similar range for end-century. The ensemble maximum projected change ranges from 0 m in mid-century to -0.03 to 0 m by end-century. Winter, Spring, and Summer ensemble maximums project an increase in minimum significant wave height of 0 to 0.03 m or 0 to 0.06 m.

All maps for RCP 8.5 projected changes in minimum and maximum significant wave heights, as well as those for 5th, 25th, 50th, 75th, and 95th percentile significant wave heights for all seasons, months, and annual timeframes can be found in Appendix K. Corresponding maps for RCP 4.5 projections can be found in Appendix F.

Figure 4-1 Annual minimum significant wave height projected changes.

Figure 4-2 Winter minimum significant wave height projected changes.

Figure 4-3 Spring minimum significant wave height projected changes.

Figure 4-4 Summer minimum significant wave height projected changes.

Figure 4-5 Fall minimum significant wave height projected changes.

## Maximum Wave Heights

This section presents the ensemble projected changes in maximum wave heights for the RCP 8.5 GHG scenario for the mid-century (2046-2061) and end-century (2081-2100). As described in Section 4.0, Figures 4-6 to 4-10 each contain six maps representing ensemble minimum (top row), mean (middle row) and maximum (bottom row) projections.

A summary of values representative of ARP Pilot Area 4 from the following figures are shown in Table 4-2. Overall, the ensemble projections vary from decreases, to little change, to increases in maximum significant wave height.

Table 4-2: Projected Changes in Maximum Wave Height Summary

| Climate Change Signal (m) | Ensemble Minimum |  | Ensemble Mean |  | Ensemble Maximum |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Mid-Century | End-Century | Mid-Century | End-Century | Mid-Century | End-Century |
| Annual | -1.7 to -0.4 | -1.2 to -0.8 | -0.4 to 0.4 | 0 to 0.4 | 0.4 to 0.8 | 0 to 0.8 |
| Winter | -0.8 to 0 | -0.8 | 0 to 0.4 | 0 | 0.8 to 1.2 | 0.4 to 1.2 |
| Spring | -1.2 to 0 | -1.2 to 0 | -0.4 to 0 | -0.8 to 0 | 0 to 0.4 | 0 |
| Summer | -0.8 to 0 | -0.8 | -0.4 to 0 | -0.4 to 0 | 0 | 0 |
| Fall | -1.2 -0.4 | -1.2 to -0.8 | -0.4 to 0 | -0.4 | 0 to 0.4 | 0 to 0.8 |

The ensemble minimum projected change in maximum annual significant wave height was found to range from -1.7 to -0.4 m in mid-century to -1.2 to -0.8 m by end-century. The ensemble mean projected little change, favouring a slight increase by end-century. The ensemble maximum projected an increase on the order of 0 to 0.8 m in the maximum annual significant wave height by end-century.

Similar to Section 4.1, the projected changes in seasonal maximum significant wave heights follow a similar pattern as the annual changes. Spring, Summer, and Fall exhibit similar projected changes ranging from lowest ensemble minimum values of -1.2 to -0.8 m up to ensemble maximums of 0 to 0.4 m or 0 to 0.8 m. Winter showed the largest projected increase with ensemble maximum values ranging from 0.8 to 1.2 m by mid-century.

Projected changes in the median (50th percentile) significant wave height follow a similar pattern as changes in the minimum and maximum significant wave heights. By end-century, the ensemble minimum projects a decrease of -0.17 to -0.06 m, the ensemble mean projected change is around -0.06 m, and the ensemble maximum shows an increase of 0 to 0.6 m.

All maps for RCP 8.5 projected changes in minimum and maximum significant wave heights, as well as those for 5th, 25th, 50th, 75th, and 95th percentile wave heights for all seasons, months, and annual timeframes can be found in Appendix K. Corresponding maps for RCP 4.5 projections can be found in Appendix F.

Figure 4-6 Annual maximum significant wave height (HsAx) projected changes.

Figure 4-7 Winter maximum significant wave height projected changes.

Figure 4-8 Spring maximum significant wave height projected changes.

Figure 4-9 Summer maximum significant wave height projected changes.

Figure 4-10 Fall maximum significant wave height projected changes.

## Extreme Wave Height Indices

This section presents the ensemble projected changes in extreme wave height indices (as defined in Table 2-1) for the RCP 8.5 GHG scenario for the mid-century (2046-2061) and end-century (2081-2100). As described in Section 4.0, Figures 4-11 to 4-15 each contain six maps representing ensemble minimum (top row), mean (middle row) and maximum (bottom row) projections.

A summary of values representative of ARP Pilot Area 4 from the following figures are shown in Table 4-3. Overall, extreme wave conditions are projected to decrease in magnitude.

Table 4-3: Projected Changes in Extreme Wave Height Indices Summary

| Extreme Wave Height Indices* | Ensemble Minimum |  | Ensemble Mean |  | Ensemble Maximum |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Mid-Century | End-Century | Mid-Century | End-Century | Mid-Century | End-Century |
| fHsRo (% pts) | -2.8 to -0.7 | -3.5 to -1.4 | -1.4 to 0 | -2.1 to -1.4 | -0.7 to 0.7 | -1.4 to 0.7 |
| fHsHi (% pts) | -0.3 to 0 | -3.6 to -1.5 | -0.3 to 0 | -0.3 to 0 | 0 to 0.3 | -0.3 to 0 |
| fHs90p (% pts) | -0.025 to 0 | -0.025 to 0 | -0.004 to 0.005 | 0 to 0.005 | 0 to 0.018 | 0.005 to 0.023 |
| fHs10p (% pts) | -0.045 to -0.015 | -0.045 to -0.015 | -0.015 to 0.015 | -0.015 to 0.015 | 0 to 0.09 | 0 to 0.09 |
| HHsDI (days) | -2.0 to -0.2 | -1.3 to 0 | 0 to 0.2 | 0.2 to 0.75 | 0.75 to 1.8 | 1.0 to 1.8 |

* fHsRo - Frequency of rough wave days (Hs > 2.5 m)
  fHsHi - Frequency of high wave days (Hs > 6 m)
  fHs90p - Frequency of top decile wave days
  fHs10p - Frequency of low decile wave days
  HHsDI - Top decile wave spell duration indicator

The ensemble mean projected change in the frequency of rough wave days (fHsRo, Figure 4-11), when the daily maximum Hs is greater than 2.5 m, is -2.1 to -1.4 percentage points by end century. The ensemble minimum and maximum projected changes are -3.5 to -1.4 and -1.4 to 0.7 percentage points respectively for end-century.

As a point of reference, a change in one percentage point corresponds to just over 3.6 days. One day is roughly 0.27% while one week is around 1.9%.

The frequency of high wave days (fHsHi, Figure 4-12), when the daily maximum Hs is greater than 6 m,  is projected to experience a slight decrease or little change, according to the ensemble mean and maximum. The ensemble minimum is projecting a decrease of -3.5 to -1.4 percentage points by end-century.

The annual percentage of days when daily max Hs > 90th percentile of the base period (1986–2005) Hs (fHs90p, Figure 4-13) and the annual percentage of days when daily max Hs < 10th percentile of base period Hs (fHs10p, Figure 4-14) are both projected to experience very little change. As can be seen in the corresponding figures, there is a scattering of slightly positive and slightly negative values throughout the region.

The ensemble mean projected change in the annual count of days with at least 2 consecutive days when daily max Hs > 90th percentile of base period Hs (HHsDI, Figure 4-15), is 0.2 to 0.75 days by end century, and a more modest 0 to 0.2 days by mid-century. The ensemble minimum is projecting a decrease of -2.0 to -0.2 days by mid-century while the ensemble maximum is projecting an increase of up to 1.8 days for both time horizons. So in general, while the frequency of extremely windy days is projected to decrease, the number of consecutive days that experience a top-decile wind speed is projected in increase.

All maps for RCP 8.5 projected changes extreme wave height indices can be found in Appendix L. Corresponding maps for RCP 4.5 projections can be found in Appendix G.

Figure 4-11 Frequency of rough wave days (fHsRo) projected changes.

Figure 4-12 Frequency of high wave days (fHsHi) projected changes.

Figure 4-13 Frequency of top decile wave days (fHs90p) projected changes.

Figure 4-14 Frequency of low decile wave days (fHs10p) projected changes.

Figure 4-15 Top decile wave spell duration indicator (HHsDI) projected changes.

## Minimum Wind Speeds

The ensemble mean projected changes in annual minimum wind speed are close to 0 m/s for mid and end-century horizons. Also for both horizons, the ensemble minimum indicates there could be decrease on the order of -0.1 m/s while the ensemble maximum shows the potential for a slight increase on the order of 0.1 m/s. Figure 4-16 gives more detail on the regional ensemble climate projections for minimum annual wind speeds.

The ensemble mean projected changes in winter minimum wind speed is relatively close to 0 m/s for mid and end-century horizons. The ensemble minimum projects a decrease of around -0.1 to -0.25 m/s for mid-century, while the end-century projected changes are predominantly -0.25 m/s. The ensemble maximum shows a projected increase of 0.15 m/s through to end-century. See Figure 4-17 for more details. Projected changes for spring and summer are similar to those for winter, with ensemble minimum and maximum results slightly closer to zero, as in Figures 4-18 and 4-19, respectively.

Ensemble mean projected changes in fall minimum wind speed show a decrease on the order of -0.1 m/s for some parts of Area 4 for mid-century with a wider prevalence of that projected decrease for end-century. The mid-century ensemble minimum projection shows a changes of -0.15 to 0 m/s with the end-century showing -0.15 to -0.1 m/s. The mid and end-century ensemble maximum projections show small increases of 0.1 m/s.

Table 4-4 presents a summary of the projected changes in minimum wind speed for the figures presented in this section.

Table 4-4: Projected Changes in Minimum Wind Speed Summary

| Climate Change Signal (m/s) | Ensemble Minimum |  | Ensemble Mean |  | Ensemble Maximum |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Mid-Century | End-Century | Mid-Century | End-Century | Mid-Century | End-Century |
| Annual | -0.1 | -0.1 | -0.4 to 0.4 | 0 to 0.4 | 0.1 | 0.1 |
| Winter | -0.25 to -0.1 | -0.25 | 0 | 0 | 0.15 | 0.15 |
| Spring | -0.1 to 0 | -0.1 | 0 to 0.1 | -0.1 to 0 | 0.1 | 0 to 0.1 |
| Summer | -0.15 to -0.1 | -0.15 to -0.1 | -0.1 to 0.1 | -0.1 to 0 | 0 to 0.1 | 0.1 |
| Fall | -0.15 to 0 | -0.15 to -0.1 | -0.1 to 0 | -0.1 to 0 | 0 to 0.1 | 0.1 |

All maps for RCP 8.5 projected changes in minimum and maximum wind speeds, as well as those for 5th, 25th, 50th, 75th, and 95th percentile wind speeds for all seasons, months, and annual timeframes can be found in Appendix M. Corresponding maps for RCP 4.5 projections can be found in Appendix H.

Figure 4-16 Annual minimum wind speed projected changes.

Figure 4-17 Winter minimum wind speed projected changes.

Figure 4-18 Spring minimum wind speed projected changes.

Figure 4-19 Summer minimum wind speed projected changes.

Figure 4-20 Fall minimum wind speed projected changes.

## Maximum Wind Speeds

The end-century ensemble projected changes in annual maximum wind speed (WsAx) range from an ensemble minimum of approximately -7.5 m/s (-6.0 m/s for mid-century) to an ensemble maximum increase of 1.5 m/s (similar for mid-century). The ensemble mean change is around -1.5 m/s for mid and end-century horizons. See Figure 4-21 for more details.

The ensemble mean fall and winter maximum wind speed changes range from -1.5 to 0 m/s for mid and end-century. The ensemble minimum projected changes are in the range of -3.5 to -1.5 m/s for mid and end-century (slightly less for fall end-century). The ensemble maximum projected changes for winter are predominantly on the order of 1.5 m/s for both horizons, no change is projected in the fall. See Figures 4-22 and 4-25 for more details on winter and fall projections, respectively.

As per Figure 4-23, the ensemble minimum, mean and maximum projected change in spring maximum wind speed are -3.5 to -2.5 m/s, -1.5 m/s, and 0 m/s, respectively, for both time horizons. Summer ensemble minimum changes (Figure 4-24) are -3.5 to -1.5 m/s for mid and end-century. Ensemble mean indicates no projected change for mid-century and a -1.5 m/s change for end century, the ensemble maximum has no projected change for end-century but an increase of 1.5 m/s for mid-century.

Table 4-5 presents a summary of the projected changes in maximum wind speed for the figures presented in this section.

Table 4-5: Projected Changes in Maximum Wind Speed Summary

| Climate Change Signal (m/s) | Ensemble Minimum |  | Ensemble Mean |  | Ensemble Maximum |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Mid-Century | End-Century | Mid-Century | End-Century | Mid-Century | End-Century |
| Annual | -6.0 | -7.5 | -1.5 | -1.5 | 1.5 | 1.5 |
| Winter | -3.5 to -1.5 | -3.5 to -1.5 | -1.5 to 0 | -1.5 to 0 | 1.5 | 1.5 |
| Spring | -3.5 to -2.5 | -3.5 to -2.5 | -1.5 | -1.5 | 0 | 0 |
| Summer | -3.5 to -1.5 | -3.5 to -1.5 | 0 | -1.5 | 1.5 | 0 |
| Fall | -2.5 to -1.5 | -1.5 | -1.5 to 0 | -1.5 to 0 | 0 | 0 |

All maps for RCP 8.5 projected changes in minimum and maximum wind speeds, as well as those for 5th, 25th, 50th, 75th, and 95th percentile wind speeds for all seasons, months, and annual timeframes can be found in Appendix M. Corresponding maps for RCP 4.5 projections can be found in Appendix H.

Figure 4-21 Annual maximum wind speed (WsAx) projected changes.

Figure 4-22 Winter maximum wind speed projected changes.

Figure 4-23 Spring maximum wind speed projected changes.

Figure 4-24 Summer maximum wind speed projected changes.

Figure 4-25 Fall maximum wind speed projected changes.

## Extreme Wind Speed Indices

No members of the ensemble project any change in the frequency of calm wind days (fWsB0) for either time horizon, as shown in Figure 4-26.

The ensemble mean and minimum both project decreases in the annual percentage of days when daily max Ws > 14.4 m/s (Beaufort Scale 7) (fWsB7, Figure 4-27).The ensemble minimum change for mid and end century are -2.0 to -4.0 and -3.0 to -6.0 percentage points respectively. Correspondingly the ensemble mean changes are -1.0 to -2.0 and -2.0 to -3.0 percentage points, respectively. The ensemble maximum projects no change for mid-century and a change of 0 to -1.0 percentage points for end-century.

Projected changes are very similar to the annual percentage of days when daily max Ws > 17.2 m/s (Beaufort Scale 8) (fWsB8, Figure 4-28). The ensemble minimum projected changes range from -2.0 to -4.0 and -2.0 to -5.0 percentage points for mid and end-century respectively. The ensemble mean projected changes are relatively consistent across both time horizons with decreases of around -1.0 percentage points. The ensemble max is projecting no change.

The ensemble maximum is also projecting no change for in the annual percentage of days when daily max Ws > 20.8 m/s (Beaufort Scale 9) (fWsB9, Figure 4-29), and the ensemble mean is projecting a small decrease of -1.0 to 0 percentage points for both time horizons. The ensemble minimum projected change is -2.5 to -1.0 for mid and end-century.

The ensemble maximum is also projecting no change for in the annual percentage of days when daily max Ws > 24.7 m/s (Beaufort Scale 10) (fWsB10, Figure 4-30), and the ensemble mean projected change is between 0 and -0.5 percentage points for both time horizons. The Ensemble minimum projected change is -0.5 to -2.0 percentage points for both time horizons.

The annual percentage of days when daily max Ws > 90th percentile of base period (1986–2005) (fWs90p, Figure 4-31) is not projected to change by much. The projected changes across the entire ensemble ranges from -0.002 to 0.002 percentage points. A similar story can be told for the annual percentage of days when daily max Ws < 10th percentile of base period (fWs10p, Figure 4-32). The full ensemble range of projected changes is -0.003 to 0.015 percentage points.

The ensemble mean projected changes in annual count of days with at least 2 consecutive days when daily max Ws > 90th percentile of base period (HWsDI, Figure 4-33) are from -1.0 to 1.0 days for both time horizons. The ensemble minimum changes range from -3.0 to -2.0 days for both time horizons, the ensemble maximum changes range from 1.0 to 2.5 and 1.5 to 2.0 days for mid and end-century respectively.

The annual count of days with at least 2 consecutive days when daily max Ws < 10th percentile of base period (LWsDI, Figure 4-34) also has a broad range of projected changes. The ensemble mean changes range from -0.75 to 0.5 and -0.5 to 0.75 days for mid and end-century, respectively. The ensemble minimum projected changes are -0.75 to -3.0 and -1.5 to -4.0 days for the respective time horizons. The ensemble maximum projections range from 1.5 to 2.5 and 1.5 to 3.0 days for the respective time horizons.

Table 4-6 presents a summary of the projected changes in extreme wind speed indices for the figures presented in this section.

Table 4-6: Projected Changes in Extreme Wind Speed Indices Summary

| Extreme Wind Speed Indices* | Ensemble Minimum |  | Ensemble Mean |  | Ensemble Maximum |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Mid-Century | End-Century | Mid-Century | End-Century | Mid-Century | End-Century |
| fWsB0 (% pts) | 0 | 0 | 0 | 0 | 0 | 0 |
| fWsB7 (% pts) | -4.0 to -2.0 | -6.0 to -3.0 | -2.0 to -1.0 | -3.0 to -2.0 | 0 | -1.0 to 0 |
| fWsB8 (% pts) | -4.0 to -2.0 | -5.0 to -2.0 | -1.0 | -1.0 | 0 | 0 |
| fWsB9 (% pts) | -1.0 to -2.5 | -1.0 to -2.5 | -1.0 to 0 | -1.0 to 0 | 0 | 0 |
| fWsB10 (% pts) | -2.0 to -0.5 | -2.0 to -0.5 | -0.5 to 0 | -0.5 to 0 | 0 | 0 |
| fWs90p (% pts) | -0.001 to 0 | 0 to 0.001 | 0 | 0.001 | 0.001 | 0.002 |
| fWs10p (% pts) | -0.001 | 0 to 0.001 | 0 to 0.001 | 0.002 to 0.004 | 0.001 to 0.002 | 0.002 to 0.015 |
| HWsDI (days) | -3.0 to -2.0 | -3.0 to -2.0 | -1.0 to 1.0 | -1.0 to 1.0 | 1.0 to 2.5 | 1.5 to 2.0 |
| LWsDI (days) | -3.0 to -0.75 | -4.0 to -1.5 | -0.75 to 0.5 | -0.5 to 0.75 | 1.5 to 2.5 | 1.5 to 3.0 |

* fWsB0 - Frequency of calm wind days (Ws < 0.5 m/s)
fWsB7 - Frequency of near gale-force wind days (Ws > 14.4 m/s)
fWsB8 - Frequency of gale-force wind days (Ws > 17.2 m/s)
fWsB9 - Frequency of strong gale-force wind days (Ws > 20.8 m/s)
fWsB10 - Frequency of storm-force wind days (Ws > 24.7 m/s)
fWs90p - Frequency of top decile wind days 
fWs10p - Frequency of low decile wind days 
HWsDI - Top decile wind spell duration indicator 
LWsDI - Low decile wind spell duration indicator

All maps for RCP 8.5 projected changes extreme wind indices can be found in Appendix N. Corresponding maps for RCP 4.5 projections can be found in Appendix I.

Figure 4-26 Frequency of calm wind days (fWsB0) projected changes.

Figure 4-27 Frequency of near gale-force wind days (fWsB7) projected changes.

Figure 4-28 Frequency of gale-force wind days (fWsB8) projected changes.

Figure 4-29 Frequency of strong gale-force wind days (fWsB9) projected changes.

Figure 4-30 Frequency of storm-force wind days (fWsB10) projected changes.

Figure 4-31 Frequency of top decile wind days (fWs90p) projected changes.

Figure 4-32 Frequency of low decile wind days (fWs10p) projected changes.

Figure 4-33 Top decile wind spell duration indicator (HWsDI) projected changes.

Figure 4-34 Low decile wind spell duration indicator (LWsDI) projected changes.

## Monthly Wind Direction

In this section the projected changes in mean monthly wind direction are presented. Figures 4-35 through 4-46 (January through December, respectively) show the projected impact of climate change following the RCP 8.5 GHG scenario, as per Wang et al (2015). The mid-century time horizon change (left side of figures) is determined by subtracting the mean monthly wind direction of the future period 2046-2065 from that of the 1986-2005 base period. The end-century values (right side of figures) compared the future period 2081-2100 to 1986-2005.

For most months in ARP Pilot Area 4, very little change in wind direction was projected for both time horizons. The majority of changes fell between 22.5° anti-clockwise (light blue) and 22.5° clockwise (light red). Many of the black arrows in the figures (representing mean monthly future wind direction, blowing toward the arrow heads) overlap almost completely with the green arrows (mean monthly base period wind direction), which indicates no change in direction.

Figure 4-35 January mean wind direction projected changes.

January, February and March (Figures 4-35, 4-36 and 4-37, respectively) each showed slight anti-clockwise rotation of the mean monthly wind directions for both time horizons. May through September (Figures 4-39 to 4-43, respectively) showed virtually no projected change. October and November (Figures 4-40 and 4-41, respectively) wind directions are each projected to change up to 22.5 degrees anti-clockwise by mid-century and between 22.5 and 45° anti-clockwise by end-century. December (Figure 4.42) shows slight anti-clockwise rotation by mid-century but closer to 22.5° by end-century.

The month projecting the greatest changes for the area of interest is April (Figure 4-38). By mid-century a change of 22.5 to 45° anti-clockwise is projected, which would result in the wind blowing toward east-south-east (compared to south-east in the base period). By end-century, the change is projected to reach 45 to 67.5° anti-clockwise, resulting in the wind blowing toward east-north-east.

Figure 4-36 February mean wind direction projected changes.

Figure 4-37 March mean wind direction projected changes.

Figure 4-38 April mean wind direction projected changes.

Figure 4-39 May mean wind direction projected changes.

Figure 4-40 June mean wind direction projected changes.

Figure 4-41 July mean wind direction projected changes.

Figure 4-42 August mean wind direction projected changes.

Figure 4-43 September mean wind direction projected changes.

Figure 4-44 October mean wind direction projected changes.

Figure 4-45 November mean wind direction projected changes.

Figure 4-46 December mean wind direction projected changes.

All maps of projected changes in monthly wind direction for RCP 8.5 are also available in Appendix O. Corresponding maps for RCP 4.5 can be found in Appendix J.

## Evaluation of Simulated Wind from Climate Models

To evaluate climate model fidelity, the simulated CMIP5 climate model data was compared with CFSR reanalysis data by calculating the root mean square error (RMSE) for each grid cell for each climate model. The RMSE represents the combined contributions of spatial pattern errors, seasonal cycle errors, and low-frequency variation errors in the simulated fields, as described in Section SM3 of Wang et al. (2014). Each model was given a score, derived from the mean of each grid cell’s score weighted by grid cell area.

To calculate the RMSE ϵk,i, wind speed fields were created from CFSR and CMIP5 members for 1981-2005. For each climate model k, 5-year monthly climatologies were created from these fields for each grid cell i. The square error is given by taking the squared difference between the simulated Wsk,i,m,d and reanalysis WsR, i,m,d wind speed fields. The mean square error was calculated by taking the sum of the squared errors across each calendar month m in the 5-year period d, then dividing by the number of elements. The RMSE of the specified grid cell for a given climate model is found by taking the square root of the mean square error.

ϵk,i=d=15 m=112 Wsk,i,m,d-WsR, i,m,d25×1212

The model score represents the fidelity of the simulated climate model in comparison to the reanalysis data. The model score is the mean of each grid cell’s RMSE, weighted by grid cell area (ie. larger grid cells have a higher weight). The results for each ensemble members are shown in Figure 4-47, which includes a regional mean value located at the top right of the respective maps.

Figure 4-47 Root Mean Square Error of Simulated Historical Surface Wind speed - CMIP5 Models compared to CFSR Reanalysis.

Overall the model were all found to have relatively high and comparable fidelity, particularly in regions removed from the coastline. The highest RMSE values for all ensemble members were found near the shore, which the coarse resolution of the CMIP5 models was not able to capture as well as the higher resolution CFSR.

# Discussion of Analysis Results

The discussion of analysis results is divided into three sections. Section 5.1 will discuss the key risks and impacts for the marine transportation industry related to historical and projected future changes in significant wave heights and surface winds as well as a description of why changes in wind and wave climate (statistics and indices, as calculated above) are significant with respect to threats to the marine transportation industry. Section 5.2 presents a similar discussion for oil spill response considerations. Section 5.3 presents a qualitative, high-level description of marine climate elements (other than wave heights and surface winds) and their key risks and impacts on marine transportation and spill response.

## Key Risks and Impacts for the Marine Transportation Industry

The discussion presented in Sections 5.1.1 and 5.1.2 is based on the results of the historical analysis (Section 3) and projected future changes (Section 4). Refer to the respective sections and appendices for detailed analysis results.

Wind speed and wind direction are critical climatic variables to monitor for the marine transportation industry. Globally, wind speeds have increased in recent decades (Young et al., 2011) and increased wind speeds lead to increased wave heights. In combination, increased wind speeds and wave heights can cause damage to ships, cargo, marine facilities and coastal infrastructure.  Higher wind speeds and wave heights may affect shipping routes and maneuvering due to increased wind and hydrodynamic resistance factors. Storm frequency will likely be affected by a northward shift in storm tracks (Loder et al., 2013). The increased frequency and intensity of storms in this region could cause delays in shipping times.

### Significant Wave Height

The historical analysis results of significant wave height are presented in Sections 3.1, 3.2 and 3.3 (and associated appendices) for long-term means and linear trends of the minimum wave heights, maximum wave heights, and extreme wave height indices respectively. The corresponding projected changes out to the middle and end of the 21st century can be found in Sections 4.1 to 4.3 and associated appendices.

#### Historical Analysis

Sections 3.2 and 3.4 conclude that the annual maximum wave height (HsAx) is approximately 3 m in the Strait of Canso, 5-7 m along most of the southeast coast, and nearly 9 m in regions furthest offshore. The highest waves are in the month of January. In addition, the frequency of rough wave days (fHsRo) (daily Hs > 2.5 m) is less than 2% near the Straight of Canso, 15% along the southeast coast and up to 45% and greater further offshore, while the frequency of high wave days (daily Hs > 6.0 m) (fHsHi) is 0% near the Strait of Canso and along other parts of the coast, and over 3% further offshore.

The linear trends analysis shows that the increase of the annual maximum wave heights near the coast are insignificant through the year, while increase at a rate of 0.2 - 0.3 m per decade in most offshore regions.

In general, marine safety is affected mainly by environmental conditions (winds, current and waves) when vessels arrive and depart a port or terminal, as well as berthing and loading/unloading at berth. The wave height limits for safe navigation, manoeuvring and marine operations recommended by the World Association of Waterborne Infrastructure (PIANC) are Hs ≤ 2 - 5 m for vessels travelling and manoeuvring in channel and harbour entrance, and Hs ≤ 0.3 – 2.5 m berthing and loading in basin and quay (varying with the types, sizes and orientation of vessels) (PIANC, 2012 and 2014).

Based on the above discussion, the sea states in ARP Pilot Area 4 particular in Port Hawkesbury and the Strait of Canso present favourable wave conditions for vessel navigation, manoeuvring and marine operations.

#### Projected Changes

The results of the ensemble maximum projected future changes show that the ensemble projected change in maximum annual wave height is in the range of 0.4 to 0.8 m by mid-century and 0 to 0.8 m by end-century in ARP Pilot Area 4. The ensemble maximum projected changes in seasonal maximum wave heights for spring, summer and fall are lie between 0 to 0.4 and 0 to 0.8 m, while up to 0.8 to 1.2 m for winter season by mid-century.

The historical analysis indicates that the seasonal maximum wave heights through spring to fall are less than 2.0 m. Therefore the projected changes of 0 to 0.8 m are unlikely to impact significantly on marine activities in Port Hawkesbury and the Strait of Canso in seasons of spring, summer and fall.

However, the historical analysis shows that the higher seasonal waves approach the Straight of Canso in winter with the maximum wave height (HsAx) up to 3 m. The ensemble maximum projected changes in winter season will increase the wave heights in Port Hawkesbury and the Strait of Canso. This will raise risk of shipping delays and potential damage for marine operations as the winter waves may exceed the operational limits (Hs ≤ 2 - 5 m in channel and harbour entrance and Hs ≤ 0.3 – 2.5 m in basin and quay) and further result in decreasing time for ship manoeuvring and loading/unloading at terminal and increasing berthing time at terminal and delaying departure. Also marine facilities and coastal infrastructure may be subject to greater damage from those winter high waves.

### Surface Wind Speed and Direction

The historical analysis results of surface wind speed are presented in Sections 3.4, 3.5 and 3.6 (and associated appendices) for long-term means and linear trends of the minimum wind speeds, maximum wind speeds, and extreme wind speed indices respectively. The corresponding projected changes out to the middle and end of the 21st century can be found in Sections 4.4 to 4.6 and associated appendices. Historical and climate change projections of wind direction can be found in Sections 3.7 and 4.7, respectively.

#### Historical Analysis

According to Sections 3.5 and 3.6, the annual maximum wind speed (WsAx) ranges approximately from 22 m/s near the Straight of Canso up to 25 m/s in eastern offshore regions. The annual percentage of days when daily max Ws > 14.4 m/s is approximately 16.5% near the Straight of Canso.

Highest monthly maximum wind speed (WsMx) occur in offshore regions of January and December (22 m/s), while July sees only 13 m/s in offshore. In western and sheltered areas July’s values are as low as 12 m/s while December and January experience approximately 20 m/s.

The analysis also shows that the trend of the annual maximum wind speed is slightly upwards at a rate less than 0.1 m/s per decade in eastern regions and slightly downwards approaching -0.03 m/s per decade in isolated southern regions.

As aforementioned marine transportation is affected mainly by environmental conditions near port or terminal. As per PIANC, the operational wind speed limits are Ws,1min ≤ 10 to 15 m/s (proportional to Ws,1hour ≤ 8.6 to 12.5 m/s) in channel and harbour entrance, and Ws,1min ≤ 10 to 30 m/s (proportional to Ws,1hour ≤ 8.6 to 23.1 m/s) in basin and quay (varying with the types, sizes and orientation of vessels). Ws,1min (Ws,1hour ) denotes 1-minite (1-hour) average wind speed at a height of 10 m above sea level.

As per the discussion above, winds during winter season are a key risk factor for marine transportation in ARP Pilot Area 4 because the wind speed may exceed the operational limits and further result in shipping delays and vessel damage.

#### Projected Changes

As per Table 4-5, the ensemble projected changes in annual maximum wind speed (WsAx) is 1.5 m/s by both mid-century and end-century. Similarly, the ensemble maximum projected change in winter maximum wind speed is 1.5 m/s for both time horizons. Summer ensemble maximum has no projected change for end-century but an increase of 1.5 m/s for mid-century.

As the summer monthly maximum wind speed is 12 m/s in the sheltered area in the Straight of Canso, an increase in summer maximum wind speed of 1.5 m/s is not considered to be an issue with marine transportation. However the high wind speeds occur mostly in winter (approximately maximum 20 m/s near the Straight of Canso), the ensemble maximum projected future increase in maximum wind speed (1.5 m/s) could lead to more downtime for marine transportation and produce higher risk of vessel damage in winter season.

In addition, the projected changes in monthly wind direction mostly occur in summer season thus impact on marine transportation due to wind direction changes is not considered to be an issue with this area.

## Key Risks and Impacts for Oil Spill Response

This goal of this section is to discuss this study’s results for the provision of operational science to support oil spill emergency response and preparedness. The focus of the discussion is on the marine climate elements that could affect the fate and behavior of an oil spill and influence response and recovery efforts and is based on the results of the historical analysis (Section 3) and projected future changes (Section 4).

Upon release into the marine environment oil will be subject to weathering processes, including spreading, evaporation, dissolution, dispersion of oil droplets into the water column, photochemical oxidation, emulsification, microbial degradation, adsorption onto particulate matter, ingestion by aquatic organisms, sinking and sedimentation. Winds and waves are key factors in these processes and also influence response efforts for containment and recovery of the spilled oil.

Immediately after the oil is spilled, it starts spreading over the sea surface, at a rate that is largely dependent on the viscosity (resistance to flow) of the oil. Viscosity is affected by the pour point - the lowest temperature at which the oil will flow. At temperatures below the pour point, the fuel solidifies rapidly and spreading is minimized. Pour point also affects dispersibility. The rate of spreading also depends on the wind speed, wave height, and the presence of tidal and mean currents.

Sections 5.2.1 and 5.2.2 discuss the role of waves and winds in spill fate and response together with observations on the potential utility of some of this study’s wave and climatology analyses for spill response efforts.

In Section 5.3 some general discussion is provided on the role of other climate elements such as temperature and ice in spill fate and response.

### Significant Wave Height

Increased wind waves and swell introduce safety risks to personnel involved in spill response efforts. High seas will limit the windows of opportunity to employ mechanical containment booms, apply chemical dispersants or conduct in situ burning. As seas rise, uneven surface areas in a containment boom hamper the boom’s effectiveness in directing or containing floating oil; oil can also splash over the boom. There is added risk of equipment damage. For personnel offshore and equipment onshore the presence of large, unpredictable waves introduce other hazards.

In addition to the expected wave climate for the area, including the analysis products prepared in this study, it is important to note that actual conditions will be affected by local weather and it is important for responders to make use of reliable local knowledge when available. For example, the effects of shoaling and exposure to large seas along much of the Eastern Shore and Cape Breton coastlines need to be considered in any planning and response efforts. Additional considerations are discussed as part of the role of winds in oil spill response in Section 5.2.2.

#### Historical Analysis

Two wave analysis products from this study are selected for discussion: the 50th percentile Hs, which reports median sea conditions; and fHsRo, the annual frequency of daily maximum Hs over 2.5 m. The fHsRo index is selected given its threshold of 2.5 m being close, albeit a bit above, the 1 to 2 m threshold generally considered for successful deployment and operation of mechanical booms for spill response in open water.

To provide an overview of the range of median conditions, the maps of seasonal 50th percentile Hs, long-term mean for 1954-2015 (Appendix A) are considered. In winter, nearshore, e.g., Chedabucto Bay and along the Eastern Shore to Cape Breton, values range from 0.64 to 1.5 m, and progressively increase as one moves offshore reaching 2.74 m farthest offshore to the southeast. In spring, nearshore values are as low as 0.6 to 1.0 m and are as large as 1.77 m offshore. Conditions are even more favourable in summer: nearshore 50th percentile Hs values are 0.3 to 1.0 m and reach as high as 1.25 m offshore. By fall, values have increased again, up to 0.64 to 1.25 m along the coast nearshore and are as high as 2 m offshore. For a threshold of 1-2 m for successful boom operation, this indicates favourable conditions could be expected over the area about half of the time.

Considering the linear trends for the seasonal 50th percentile Hs values, the magnitude of changes are very small and are the lowest for spring through fall. During this period, the trends range from 0.012 m/decade nearshore to 0.032 in summer, 0.53 in fall and 0.095 m/decade in spring farther offshore. In winter, trends range from 0.012 to 0.095 m/decade nearshore and also offshore with the largest values being about 0.179 m/decade along the Eastern Shore. There are no points of significance in the linear trends.

Considering fHsRo, a lower frequency of rough wave days implies more times with conditions which might be favourable for spill response. The historical long term mean and linear trend for fHsRo are presented in Appendix B. Values of fHsRo increase uniformly from about 5 to 20 % nearshore to about 40% or above offshore. The linear trends indicate increases in fHsRo of 0.5 to 1.8 % / decade over most of the area with highest values along the Eastern Shore, i.e., another 2 to 7 days per decade with daily maximum Hs over 2.5 m. These increases are reported as significant for a bit over half of the area grid cells.

#### Projected Changes

The same two wave analysis products considered above for historical analysis are assessed for the future: the 50th percentile Hs and the fHsRo index.

The projected changes for the 50th percentile Hs are presented in Appendix K. For winter, the changes in 50th percentile Hs between both mid- and end-century projections and the base period are small. The ensemble minimum change (i.e., the lowest, most negative change projected by any of the six individual models) ranges from -0.082 to 0.028 m for both mid- and end-century changes. Ensemble maximum change ranges from -0.027 to 0.083 m with the larger values located along the Eastern Shore for the mid-century comparison and still smaller changes nearshore for the end-century comparison.

For spring, the ensemble change limits range from -0.137 m for mid-century and -0.192 m for end-century ensemble minimum changes up to 0.083 m for the ensemble maximum change for mid-century.

The limits of change in summer range from -0.082 m for mid-century and -0.137 m for end-century ensemble minimum changes up to 0.083 m for the ensemble maximum change from mid-century and 0.193 m for some grid cells for end-century.

For fall, the limits of change have the greatest range of any season, from -0.192 m for mid-century and as large as -0.302 m for end-century ensemble minimum. For fall the ensemble maximum changes, are not as large, up to -0.082 m for mid-century and -0.137 m for end-century.

In all seasons, the ensemble mean changes are small at -0.027 to 0.028 m with some grid cells offshore in fall reporting as large a change as -0.137 m for both mid- and end-century.

Overall, it can be concluded that projected changes in the 50th percentile Hs for most of the Pilot Area are on the order of +0.1 m or less, with some locations, particularly for fall, being as large as -0.2 or -0.3 m. Changes in median wave conditions of this magnitude are unlikely to measurably affect spill response planning or success, the exception being for those relatively few grid cell locations and times of year if waves are on average 30 cm less which could be of benefit.

The annual frequency of days with waves of Hs ≥ 2.5 m, fHsRo, are presented in Appendix L. The projected changes in fHsRo are generally on the order of a reduction of several percent. The ensemble minimum change is about -3% (less for nearshore) for mid-century, and generally -3% and up to -4% for some grid cells farther offshore for the change in end-century compared with the base period. The ensemble mean change in fHsRo for mid-century is +1/3 % for most of the Pilot Area and in some places as large as -1.7%. For the end-century, the changes are typically -1.0 to -1.7%. The ensemble maximum changes are similar for both mid- and end-century at -1/3 to 1%. Overall, these projected changes are so small in the frequency of time the waves are exceeding an Hs of 2.5 m that by themselves they do not suggest any measurable change to the windows of opportunity for conducting spill response.

### Surface Wind Speed and Direction

Winds are a primary factor for oil spill spreading, they increase evaporation, and generate wind waves and currents. Wind speeds will limit the effectiveness of spill response options such as mechanical recovery such as booming, in situ burning or use of chemical dispersants. Typical windows of opportunity for response are as follows:

Mechanical cleanup and burning: wind speeds up to about 7 m/s (between Beaufort Force (BF) 3 and 4)

Dispersants: wind speeds up to about 13 m/s (BF 6)

Wind direction will influence which coastal areas are at risk, and changes in direction may require repositioning or redeployment of booms to maintain containment or recovery effectiveness.

Wind information is also required for spill trajectory models, a primary tool used in spill response to estimate the marine and coastal areas potentially affected as a result of a spill, together with the initial weathering fate of the fuel, e.g., change of volume of the spill over time.

Spill models rely on present observations and forecasts of environmental conditions, primarily wind and current. This study’s wind and wave climatology maps are therefore not of direct use for spill trajectory modelling; however, they have potential benefit for spill response planning. Indication of the seasonal or monthly wind and wave conditions likely to be encountered for a given response area, e.g., ARP Pilot Area 4, can assist in preparedness, planning and logistics and training efforts. As noted briefly above, wind and sea conditions define the typical windows of response. In this way, several of the study maps provide a general indication of conditions over the ARP Pilot Area 4.

#### Historical Analysis

The seasonal 50th percentile wind speed, which reports median wind speed (maps in Appendix C),  ranges from about 5 to 6 m/s in summer to 9 to 11 m/s in winter with gradual, uniform increases of 2 to 3 m/s each season as one moves from west to east, nearshore to offshore. These conditions would be generally favourable for most spill response efforts (e.g., thresholds on the order of 10 m/s noted above). As noted above, and shown in the monthly wind direction maps of Appendix E, winds over the ARP Area 4 are predominantly from the southwest in spring and summer moving to westerlies in the fall and then from the west-northwest in winter. This indicates under average wind conditions the effect of spill drift due to wind would be towards offshore.

#### Projected Changes

The project mid- and end-century predictions for seasonal 50th percentile, or median, wind speed, are reported in Appendix M. Very little changes are projected. For mid-century compared with the historical base period, ensemble minimum changes in 50th percentile wind speed are on the order of -0.5 to -0.8 m/ for all four seasons. Ensemble mean changes are +- 0.16 m/s for winter through summer, with a change up to -0.5 m/s predicted for fall. The ensemble maximum change in 50th percentile wind speed are generally slightly positive with values ranging from -0.16 to 0.5 m/s for fall through spring and up to 0.8 m/s in summer.

For projected changes to end-century the values are very similar, typically -0.5 to -0.8 m/s for ensemble minimum change, -0.5 m/s for ensemble mean change, and between -0.5 and 0.5 m/ for ensemble maximum change. None of these projected changes would have a bearing on spill response.

As reported in Section 4.7, very little change in wind direction is projected for both time horizons. Winter shows slight anti-clockwise rotation of the mean monthly wind directions while late spring through early fall show virtually no projected changes. The largest projected changes are in October and November with wind directions projected to change up to 22.5°anti-clockwise (i.e., more from the west-southwest as opposed to the historical or base period mean westerlies) by mid-century and between 22.5° and 45° anti-clockwise by end-century, and also in April. In April changes in wind direction of between 22.5° and 45°anti-clockwise are projected for mid-century, with larger shifts of 45° to 67.5°anti-clockwise projected for mid-century, which would result in winds blowing more from the west or west-southwest. For those few months with some change in wind direction the tendency is for increased frequency of winds more from the southwest. In the case of an oil spill this might mean increased likelihood of drift tangential to the Nova Scotia shoreline.

To improve the potential utility of future operational science data products, such as this report, it is recommended that response agencies and other stakeholders should agree which thresholds or indices might be most useful. For example, a fWsB4 or fWB5 for annual frequency of moderate or fresh breeze (Beaufort) force wind days would likely offer more practical guidance than frequency of high winds for which response efforts are not safe or practical to undertake or are ineffective.

## Potential Impact of other Marine Climate Elements

This section presents a qualitative, high-level description of marine climate elements (other than wave heights and surface winds) and their key risks and impacts on marine transportation and spill response.

The Port Hawkesbury and the Strait of Canso Pilot Area is located on the southeast coast of Nova Scotia. Climate change will influence a wide variety of climatic variables in this area, which in turn, will impact resource sectors and industries that rely heavily on marine resources and transportation. The climate variables discussed below include air temperature, sea surface temperature, fog, extreme winds, extreme waves, changes in wind direction, sea levels and storm surge, tides and currents, coastal profile characterization, sea ice, land-fast ice and possibility of icebergs. Winds and waves are the primary focus of this report due to their predominant influence on marine transportation and oil spill response. As such, a thorough quantitative climate change analysis is presented in Section 4 and accompanying discussion in Sections 5.1 and 5.2. That said, the climatic variables mentioned above also influence marine transportation and are the focus of the present section.

Much of the information below was found in Lemmen et al. (2016), a Government of Canada report entitled “Canada’s Marine Coasts in a Changing Climate,” and the corresponding references therein. Lemmen et al. report provides a broad and recent literature review pertinent to the scope of this section.

### Sea Surface Temperature and Air Temperature

As sea surface temperatures and air temperatures are closely related and can have similar affects, a combined discussion is presented here.

Figure 5-1 shows the seasonal sea surface temperature (SST) climatology (1981-2010) for Atlantic Canada. The plots were created using NOAA’s Optimum Interpolation (OI) SST data via the Earth System Research Laboratory’s (ESRL) online Monthly/Seasonal Climate Composites tool. (Values over land are an artifact of the interpolation scheme and should be ignored.) SSTs average 1-2°C in winter, 1-2°C in spring, 13-14°C in summer, and roughly 12°C in autumn.

Figure 5-2 shows the seasonal air temperature climatology (1981-2010) for Atlantic Canada. The plots were created using NCEP-NCAR Reanalysis data via the Earth System Research Laboratory’s (ESRL) online Monthly/Seasonal Climate Composites tool. Historical winter air temperatures average -2°C, spring temperatures 2-4°C, summer 14-15°C, while autumn air temperatures average roughly 10°C.

An increase in air temperature is the strongest climate trend and is one of the most immediate and noticeable effects of climate change. The Canadian Atlantic region has experienced a statistically significant increase in mean annual air temperature for the period 1900-2010. Air temperatures at stations located along the Atlantic Ocean increased by 0.75 ±0.34°C over this period (Ouranos, 2010). A 0.89°C increase in surface water temperature was observed for the Scotian Shelf during the period 1982-2006 (Sherman et al., 2009). Global-climate projections indicate widespread warming of 1 to 3°C by 2100 of Canada’s oceans during the 21st century, under an intermediate-emissions scenario (Meehl et al., 2007).

Figure 5-1 Sea Surface Temperature 1981-2010 Climatology. DJF (top-left), MAM (top-right), JJA (bottom-left), SON (bottom-right).

Rising air and sea surface temperatures will lead to shorter sea ice seasons. The average annual sea-ice cover on Canada’s east coast has decreased by 0.27% per year since the Canadian Ice Service began collecting data circa 1968 (Senneville et al., 2014). These trends are projected to continue, with modelling indicating that sea ice will be almost completely absent in most of the Gulf of St. Lawrence by 2100 (Senneville et al., 2014). Reduced amounts of sea ice and a shorter sea ice season will influence a number of other climatic variables such as wave height. Ice cover impedes wave formation, therefore, the shortening of the sea ice season increases the total energy of waves developing in an ice-free body of water (Neumeier et al., 2013). Reduced amounts of sea-ice will improve access to harbours year round, presenting new opportunities for shipping. As shown in Figure 5-3 (available: http://iceweb1.cis.ec.gc.ca/30Atlas/page1.xhtml), the southeast coast of Nova Scotia has historically seen a low frequency of sea ice, therefore the changes caused by a shorter sea ice season will be minimal for this area.

Figure 5-2 Surface Air Temperature 1981-2010 Climatology. DJF (top-left), MAM (top-right), JJA (bottom-left), SON (bottom-right).

Warmer air and sea surface temperatures will reduce freezing spray on marine facilities which impacts marine operation. However effect of the changes are considered to be minimal for this area.

In terms of oil spill response, at cold temperatures spilled oil will be denser and more viscous than in warmer conditions. Warmer conditions will contribute to decreased viscosity (and hence increased spreading) and an increased rate and degree of evaporation.

### Sea Ice and Icebergs

Warmer air temperatures and reduced amount of sea ice will cause glaciers in Greenland and the Canadian Arctic to melt faster (Herdes et al., 2012). Increased glacial melt will have a severe impact on the rates that icebergs are discharged into the ocean. This expected increase in iceberg calving rates poses a threat to marine transportation, therefore they are carefully tracked in order to avoid collisions. However, iceberg sightings are uncommon on the southeast coast of Nova Scotia, therefore increased iceberg production rates are unlikely to cause significant changes to marine transportation in this area. Increases in melt rates from warmer seas surface and air temperatures will also decrease the likelihood of icebergs travelling as far south as ARP Pilot Area 4.

Sea ice is present occasionally in the region, typically from as early as late January through to the end of April (CIS, 2011). Figure 5-3 shows the frequency of presence of sea ice for mid-March. Ice concentrations during that time typically range from 4/10 to 8/10 but some patches up to 9-9+/10 are possible. Corresponding ice thicknesses can range from less than 10 cm (new ice) to 30-70 cm (some thin first-year ice).

Figure 5-3 Frequency of Presence of Sea Ice, Week of March 19th (Canadian Ice Service, 2011)

In addition to colder sea temperatures which will affect the weathering of an oil spill, the presence of sea ice can have numerous effects on an oil spill which will be considerations for spill trajectory prediction and tracking and for response efforts. For example, these include:

The ice can act as a physical barrier or retardant impeding the spread of fuel and provide some containment.

In closer, more concentrated pack ice or brash ice (ice in chunks up to 2 m in diameter) fuel slicks may not reach their equilibrium thickness as spreading is slowed or stopped by the presence of ice floes in close contact or by slush. This can result in a smaller, confined slick area and greater oil thicknesses compared with open water. Ice leads and deformed ice features may create pools of fuel further reducing spreading.

Ice presence will tend to dampen wind waves. With less wave energy emulsification of fuel (which reduces the rate of other weathering processes) is uncommon, usually being decreased or does not occur

Natural dispersion – the breakup of a slick into droplets of varying sizes which become mixed into the upper layers of the water column – will occur at lower rates with reduced wave conditions due to ice, even though ice floe motion can be a source of surface turbulence.

While the magnitude of changes to sea ice is unclear, with global warming, some changes to sea ice and iceberg conditions should be expected. This may include reduced sea ice extent and ice thicknesses. Any shortening of the sea ice season would remove one potential complicating factor for spill response.

### Fog

Fog is a climatic variable that is considered due to the risks that reduced visibility has on marine transport and the high frequency of fog in Nova Scotia. Particular in the northeast (between Point Michaud to Louisbourg) of the Port Hawkesbury and the Strait of Canso Pilot Area, sea fog is frequent, dense and persistent usually associated with southerly winds in late spring and early summer. Therefore sea fog is a key risk factor for marine traffic in this area.

Warmer winters can lead to increased incidence of fog. Ice decay caused by climate change introduces additional moisture and heat into the atmosphere, which can lead to more frequent cloudiness and fog (Asplin et al., 2012). This will increase risk for marine transportation in this area.

### Sea-Level Rise, Storm Surge and Currents

Sea-level rise is an important climatic variable to consider due to the amplifying effect that rising sea-levels have on other climatic variables. Sea-level rise is influenced by the thermal expansion of the surface layer of the ocean, meltwater from glaciers and ice caps, projected changes in dynamic oceanography, and other smaller sources. For the high-emissions scenario, James et al. (2014) found a median ensemble projection of sea level rise of 80 to 100 cm at 2100 (ensemble 95th percentile increases were 120 to 140 cm), relative to 1986–2005, on the Atlantic coast of Nova Scotia as shown in Figure 5-4.

Higher sea levels can leave marine facilities and coastal infrastructure vulnerable to flooding and ice loading as they can be associated with storm surges superimposed on high tides, coupled with an increase in storm frequency and intensity. Changes to ocean currents can lead to changes in the sea-surface topography and in turn, affect local relative sea levels. Sea-level rise due to the weakening of the Gulf Stream is predicted for northeastern coastal North America in the coming century (Yin et al., 2010; Yin, 2012; Church et al., 2013). Rising sea-levels as a result of climate change will cause more extreme water-level events in the future, accelerating erosion and putting marine facilities and coastal infrastructure at risk when combined with storm surge, high tides, and increased wave action. However, deeper water in harbours can also present new opportunities in the marine transportation industry by improving access for larger ships with deeper drafts.

As noted above, surface currents play a significant role in the drift of an oil spill, with slicks drifting with the same speed and direction as the current. Currents will be factors in spill response efforts as well, e.g., high currents and cross currents will introduce challenges in booming efforts, and large currents may also further limit windows of opportunity for use of chemical dispersants or burning.

As tides change there is a risk of losing oil that has been collected in booms. Knowledge of the timing of tides and nearshore bathymetry are important considerations in spill response.

As aforementioned in Section 5.3.1, projected sea ice is in a trend of reducing and almost completely absent by 2100. However as long as sea ice presents, due to sea-level rising sea ice and land-fast ice will reach higher level of marine facilities and coastal infrastructure, consequently result in higher ice loads on these structures. This may cause potential risk of failure for marine facilities and coastal infrastructure.

Figure 5-4 Projections of relative sea-level rise by the year 2100 for the ensemble median (left) and 95th percentile (right) value of the high-emissions (RCP 8.5) scenario (from Lemmen et al., 2016 after James et al., 2014)

# Summary

This study focuses on the recent climatology and projected effects of climate change on wind and waves in Area Response Plan (ARP) Pilot Area 4: Port Hawkesbury and the Strait of Canso. This work supports the Government of Canada’s ARP Initiative which aims to strengthen national marine oil spill preparedness and response.

Climatological analysis components include the calculation of descriptive statistics (minimum, maximum, 5th, 25th, 50th, 75th, and 95th percentiles) for significant wave heights and wind speeds as well as numerous extreme value indices (defined in Tables 2-1 and 2-2), for annual, seasonal and monthly timescales. Mean monthly wind directions were also investigated. The climate change signal for all statistics, indices and wind directions were calculated for two greenhouse gas emissions scenarios out to the middle (2046-2065) and end (2081-2100) of the 21st century. The minimum, mean and maximum ensemble projection results were then mapped to ensure a broad range of climate projection uncertainty was represented.

As summarized in Table 6-1, the long-term mean of minimum annual wave heights were less than 0.4 m, median annual wave heights were between 0.5 and 1.75 m, and maximum annual wave heights ranged from 3.0 to 9.0 m. Significant wave height values were generally found to increase with distance from the coastline. Minimum, median and maximum wave heights have all been found to be trending slightly upward over the past several decades.

Also in Table 6-1, the long-term mean of minimum annual wind speeds were 0.2 to 0.5 m/s, the median annual wind speeds were around 7.0 to 8.0 m/s, and the maximum annual wind speed ranged from 22.0 to 25.0 m/s. While the minimum wind speed was trending slightly upwards in some areas, the median and maximum wind speeds were experiencing slightly negative or positive trends, depending on the specific location.

Table 6-1: Historical Annual Statistics Summary

| Annual Timeframe | Wave Height |  | Wind Speed |  |
| --- | --- | --- | --- | --- |
|  | Long-term mean (m) | Significant Trend (m/decade) | Long-term mean (m/s) | Significant Trend (m/s/decade) |
| Minimum | 0.0 to 0.4 | 0.02 to 0.03* | 0.2 to 0.5 | 0.01 to 0.02* |
| Median (50th percentile) | 0.5 to 1.75 | 0.03 to 0.07 | 7.0 to 8.0 | -0.01 to 0.03 |
| Maximum | 3.0 to 9.0 | 0.2 to 0.3 | 22.0 to 25.0 | -0.03 to 0.1* |

* Significant trend only in portions of the region, see maps for more details

As summarized in Table 6-2, there was a broad range of frequency of rough wave days from 2% of days per year in sheltered coastal areas to 45% of days further offshore. The frequency of high wave days ranged from 0 to 3%. The number of consecutive days with top-decile wave heights was between 26 and 29 days. All of these indices have been trending upward in various parts of the region.

Table 6-2: Historical Extreme Wave Height Indices Summary (from Table 3-3)

| Extreme Wave Height Indices* | fHsRo | fHsHi | fHs90p | fHs10p | HHsDI |
| --- | --- | --- | --- | --- | --- |
| Long-term mean | 2 to 45 % (7.3 to 164.3 days) | 0 to 3 % (0 to 11.0 days) | 9 to 10 % (32.9 to 36.5 days) | 10 to 16 % (36.5 to 58.4 days) | 26 to 29 days |
| Significant Trend | 1 to 1.5** % pts / decade | 0.125** % pts / decade | 0.4 to 0.7** % pts / decade | -3.0 to -1.0 % pts / decade | 1.2 to 2.0** days / decade |

* fHsRo - Frequency of rough wave days (Hs > 2.5 m)
  fHsHi - Frequency of high wave days (Hs > 6 m)
  fHs90p - Frequency of top decile wave days
  fHs10p - Frequency of low decile wave days
  HHsDI - Top decile wave spell duration indicator 
** Significant trend only in portions of the region, see maps for more details

Table 6-3 shows that there were zero calm wind days. Long-term means of high wind indices, which measured frequencies of days with wind speeds greater than 14.4 m/s (52 km/hr) up to the frequency of days with wind speeds greater than 24.7 m/s (88.9 km/hr). These ranged from 14.5-21.5% down to 0.025-0.15%, respectively. Consecutive days experiencing top-decile and bottom-decile wind speeds were 25.0-26.0 and 22.0-22.5 days, respectively. No significant trends were found for any of the extreme wind indices.

The annual cycle of mean wind direction is east-south-eastward in January through March, to more eastward in April, to north-eastward in May through August, east-north-eastward in September, back to eastward in October and November, and east-south-eastward again in December. Relatively small shifts in wind direction (between 14° clockwise and anti-clockwise) were found to occur in most months, from 1980-1995 to 2000-2015. April showed clockwise changes closer to shore (up to 21°) and anti-clockwise changes further offshore (up to 14°). May exhibited the largest changes with clockwise rotation of up to 35° in offshore areas.

Table 6-3: Historical Extreme Wind Speed Indices Summary (from Table 3-4)

| Extreme Wind Speed Indices* | Units | fWsB0 | fWsB7 | fWsB8 | fWsB9 | fWsB10 | fWs90p | fWs10p | HWsDI | LWsDI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Long-term mean** | % | 0 | 14.5 to 21.5 | 4.75 to 8.75 | 0.6 to 1.9 | 0.025 to 0.15 | 9.9 to 10.0 | 9.95 to 10.0 | - | - |
|  | days | 0 | 52.9 to 78.5 | 17.3 to 31.9 | 2.2 to 6.9 | 0.1 to 0.5 | 36.1 to 36.5 | 36.3 to 36.5 | 25.0 to 26.0 | 22.0 to 22.5 |

* fWsB0 - Frequency of calm wind days (Ws < 0.5 m/s)
fWsB7 - Frequency of near gale-force wind days (Ws > 14.4 m/s)
fWsB8 - Frequency of gale-force wind days (Ws > 17.2 m/s)
fWsB9 - Frequency of strong gale-force wind days (Ws > 20.8 m/s)
fWsB10 - Frequency of storm-force wind days (Ws > 24.7 m/s)
fWs90p - Frequency of top decile wind days 
fWs10p - Frequency of low decile wind days 
HWsDI - Top decile wind spell duration indicator 
LWsDI - Low decile wind spell duration indicator
** No significant trends were found for any extreme wind indices

Tables 6-4 to 6-6 present an overview of the ensemble mean projected changes according to RCP 8.5 as presented in Section 4. The ensemble minimum and maximum projections of Section 4 and the corresponding appendices should be viewed in conjunction with the ensemble mean projections to ensure a range of potential climate change signals are considered. Projections for RCP 4.5 (a lower GHG scenario) are also available in Appendices F to J.

According to the ensemble mean projections, wave heights are generally expected to decrease moderately in the coming decades, which runs counter to the historical trends. The frequency of rough and high wave days are projected to decrease, while the number of days with consecutive top-decile wave days is projected to increase slightly.

The ensemble mean projections show only marginal changes for minimum and median wind speeds, while the maximum wind speed is projected to decrease by 1.5 m/s. The frequencies of days with high winds (i.e. greater than 14.4, 17.2, and 20.8 m/s) are projected to increase, with virtually no change in the number of calm days or those with storm-force winds. The number of consecutive days experiencing top- or bottom-decile wind speeds is projected to increase or decrease, depending on location.

Table 6-4: Ensemble Mean Projected Changes in Annual Statistics Summary

| Ensemble Mean | Wave Height (m) |  | Wind Speed (m/s) |  |
| --- | --- | --- | --- | --- |
|  | Mid-21st Century | End-21st Century | Mid-21st Century | End-21st Century |
| Minimum | -0.03 to 0 | -0.06 to 0 | -0.4 to 0.4 | 0 to 0.4 |
| Median (50th percentile) | -0.06 to 0 | -0.06 | 0 | 0 |
| Maximum | -0.4 to 0.4 | 0 to 0.4 | -1.5 | -1.5 |

Table 6-5: Ensemble Mean Projected Changes in Extreme Wave Height Indices Summary

| Extreme Wave Height Indices | fHsRo (% pts) | fHsHi (% pts) | fHs90p (% pts) | fHs10p (% pts) | HHsDI (days) |
| --- | --- | --- | --- | --- | --- |
| Mid-Century | -1.4 to 0 | -0.3 to 0 | -0.004 to 0.005 | -0.015 to 0.015 | 0 to 0.2 |
| End-Century | -2.1 to -1.4 | -0.3 to 0 | 0 to 0.005 | -0.015 to 0.015 | 0.2 to 0.75 |

Table 6-6: Ensemble Mean Projected Changes in Extreme Wind Speed Indices Summary

| Extreme Wind Speed Indices | fWsB0 (% pts) | fWsB7 (% pts) | fWsB8 (% pts) | fWsB9 (% pts) | fWsB10 (% pts) | fWs90p (% pts) | fWs10p (% pts) | HWsDI (days) | LWsDI (days) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mid-Century | 0 | -2.0 to -1.0 | -1.0 | -1.0 to 0 | -0.5 to 0 | 0 | 0 to 0.001 | -1.0 to 1.0 | -0.75 to 0.5 |
| End-Century | 0 | -3.0 to -2.0 | -1.0 | -1.0 to 0 | -0.5 to 0 | 0.001 | 0.002 to 0.004 | -1.0 to 1.0 | -0.5 to 0.75 |

According the World Association of Waterborne Infrastructure (PIANC) criteria, the sea states in ARP Pilot Area 4 particular in Port Hawkesbury and the Strait of Canso present favourable wave conditions for vessel navigation, manoeuvring and marine operations. That said, the ensemble maximum projected changes in winter show increased wave heights in the region. This would increase the risk of shipping delays and potential damage for marine operations as the wave heights may exceed operational limits. This would result in decreasing time for ship manoeuvring and loading/unloading at terminal and increasing berthing time at terminal and delaying departure. Marine facilities and coastal infrastructure may also be subject to greater damage from higher winter waves. Projected changes are unlikely to impact significantly on marine activities in Port Hawkesbury and the Strait of Canso in spring, summer and fall.

With regard to spill response capability, median sea conditions provide some insight. Considering the historical trends for the seasonal 50th percentile Hs values, the magnitude of changes are very small. For a threshold of 1-2 m for successful boom operation, results indicate favourable conditions could be expected over the area about half of the time. Projected changes in median wave conditions are unlikely to measurably affect spill response planning or success, the exception being for those relatively few grid cell locations and times of year if waves are on average 30 cm less which could be of benefit. Projected changes in the frequency of rough wave days (fHsRo) are so small that by themselves they do not suggest any measurable change to the windows of opportunity for conducting spill response.

Winter winds are a key risk factor for marine transportation in the region as wind speeds may exceed the operational limits and result in shipping delays and vessel damage. Ensemble maximum projections in winter wind speeds could lead to more downtime for marine transportation and produce higher risk of vessel damage. Projected changes in monthly wind direction mostly occur in summer meaning the impact on marine transportation due to wind direction changes will not likely be an issue.

With regard to spill response, average historical wind conditions indicate the effect of spill drift due to wind would be towards offshore. For those few months with projected changes in wind direction, the tendency is for increased frequency of winds more from the southwest. In the case of an oil spill this might mean increased likelihood of drift tangential to the Nova Scotia shoreline. None of the projected changes in wind speed would have a bearing on spill response.

With regard to other climatic variables, warmer winters can lead to increased incidence of fog. Ice decay caused by climate change introduces additional moisture and heat into the atmosphere, which can lead to more frequent cloudiness and fog. This will increase risk for marine transportation in this area due to more instances of reduced visibility.

Rising sea-levels as a result of climate change will cause more extreme water-level events in the future, accelerating erosion and putting marine facilities and coastal infrastructure at risk when combined with storm surge, high tides, and increased wave action. However, deeper water in harbours can also present new opportunities in the marine transportation industry by improving access for larger ships with deeper drafts.

In terms of oil spill response, warmer conditions will contribute to decreased viscosity (and hence increased spreading) and an increased rate and degree of evaporation. The projected decrease in the presence of sea ice, however, could remove several complicating factors for spill response.  Surface currents will continue to play a significant role in the drift of an oil spill, with slicks drifting with the same speed and direction as the current.

# Signatures

This report has been prepared for the exclusive use of Environment and Climate Change Canada. The study was conducted in accordance with the terms of reference and written requests from the client. The conclusions presented herein are based solely upon the scope of services and time and budgetary limitations described in our proposal. Any use which a third party makes of this report, or any reliance on, or decisions to be made based on it, are the responsibility of such third parties. Amec Foster Wheeler accepts no responsibility for damages, if any, suffered by any third party as a result of decisions made or actions based on this report.

We trust that this report meets your needs and are available to discuss upon request of the client.

Yours sincerely,

Amec Foster Wheeler Environment & Infrastructure,

a Division of Amec Foster Wheeler Americas Limited

| Prepared by: | Reviewed by: |
| --- | --- |
| Jonas Roberts, PhD, PEng Climate Change Consultant | Patrick Roussel, M.Eng. M.Sc. Senior Oceanographer |

# REFERENCES

Asplin, M.G., Galley, R., Barber, D.G. and Prinsenberg, S. (2012): Fracture of summer perennial sea ice by ocean swell as a result of Arctic storms; Journal of Geophysical Research: Oceans, v. 117.

Bush, E.J., Loder, J.W., James, T.S., Mortsch, L.D. and Cohen, S.J. (2014): An overview of Canada’s changing climate; in Canada in a Changing Climate: Sector Perspectives on Impacts and Adaptation, (ed.) F.J. Warren and D.S. Lemmen; Government of Canada, Ottawa, ON, p. 23–64.

Canadian Ice Service (CIS) (2011). Sea ice climatic atlas, East Coast, 1981-2010. Retrieved March 2017 from http://ec.gc.ca/Publications/default.asp?lang=En&xml=8DFED3F9-4BD6-49F3-9ACA-F9AA9F52A96D

Church, J.A., Clark, P.U., Cazenave, A., Gregory, J.M., Jevrejeva, S., Levermann, A., Merrifield, M.A., Milne, G.A., Nerem, R.S., Nunn, P.D., Payne, A.J., Pfeffer, W.T., Stammer, D. and Unnikrishnan, A.S. (2013): Sea level change; in Climate Change 2013: The Physical Science Basis; Contribution of Working Group I to the Fifth Assessment Report of the Intergovernmental Panel on Climate Change (IPCC), (ed.) T.F. Stocker, D. Quin, G.K. Plattner, M. Tignor, S.K. Allen, J. Boschung, A. Nauels, Y. Xia, V. Bex and P.M. Migdley; Cambridge University Press, Cambridge, United Kingdom and New York, New York, p. 1137–1216.

Galbraith, H., Jones, R., Park, R., Clough, J., Herrod-Julius, S., Harrington, B. and Page, G. (2002): Global climate change and sea level rise: potential losses of intertidal habitat for shorebirds; Waterbirds, v. 25, no. 2, p. 173–183.

Greenberg, D.A., Blanchard, W., Smith, B. and Barrow, E. (2012): Climate change, mean sea level and high tides in the Bay of Fundy; Atmosphere-Ocean, v. 50, no. 3, p. 261–276.

Herdes, E., Copland, L., Danielson, B. and Sharp, M. (2012): Relationships between iceberg plumes and sea-ice conditions on northeast Devon Ice Cap, Nunavut, Canada; Annals of Glaciology, v. 53, no. 60, p. 1–9.

IPCC [Intergovernmental Panel on Climate Change] (2000): Special Report on Emissions Scenarios; Working Group III, (ed.) N. Nakicenovic and R. Swart; Cambridge University Press, Cambridge, United Kingdom, 599 p.

IPCC (2014) “Climate Change 2014: Synthesis Report. Contribution of Working Groups I, II and III to the Fifth Assessment Report of the Intergovernmental Panel on Climate Change” [Core Writing Team, R.K. Pachauri and L.A. Meyer (eds.)]. IPCC, Geneva, Switzerland, 151 pp.

James, T.S., Henton, J.A., Leonard, L.J., Darlington, A., Forbes, D.L. and Craymer, M. (2014): Relative sea-level projections in Canada and the adjacent mainland United States; Geological Survey of Canada, Ottawa, Ontario, Open File 7737, 72 p.

Lemmen, D.S., Warren, F.J., James, T.S. and Mercer Clarke, C.S.L. editors (2016): Canada’s Marine Coasts in a Changing Climate; Government of Canada, Ottawa, ON, 274p.

Loder, J.W., Han, G., Galbraith, P.S., Chassé, J. and van der Baaren, A. (2013): Aspects of climate change in the northwest Atlantic off Canada; Fisheries and Oceans Canada, Dartmouth, Nova Scotia, Canadian Technical Report of Fisheries and Aquatic Sciences 3045, 190 p.

Meehl, G.A., Stocker, T.F., Collins, W.D., Friedlingstein, P., Gaye, A.T., Gregory, J.M., Kitoh, A., Knutti, R., Murphy, J.M., Noda, A., Raper, S.C.B., Watterson, I.G., Weaver, A.J and Zhao, Z.-C. (2007): Global climate projections; in Climate Change 2007: The Physical Science Basis (Contribution of Working Group I to the Fourth Assessment Report of the Intergovernmental Panel on Climate Change), (ed.) S. Solomon, D. Qin, M. Manning, Z. Chen, M. Marquis, K.B. Averyt, M. Tignor and H.L. Miller; Cambridge University Press, Cambridge, United Kingdom and New York, New York, p. 747-845.

Neumeier, U., Ruest, B., Lambert, A., Bismuth, E., Dumont, D., Jacob, D., Savard, J.-P. and Joly, S. (2013): Modélisation du régime des vagues du golfe et de l’estuaire du Saint-Laurent pour l’adaptation des infrastructures côtières aux changements climatiques; report prepared bythe Institut des sciences de la mer de Rimouski, Université du Québec à Rimouski (UQAR) for the ministère des Transports du Québec, 253 p.

Ouranos (2010): Élaborer un plan d’adaptation aux changements climatiques – Guide destiné au milieu municipal québécois; Ouranos, Montréal, Québec, 45 p.

PCIC [Pacific Climate Impacts Consortium] (2013): Climate summary for South Coast Region – part of a series on the resource regions of British Columbia; Pacific Climate Impacts Consortium, University of Victoria, Victoria, British Columbia, 4 p.

PIANC (2012) “Safety Aspects Affecting the Berthing Operations of Tankers to Oil and gas Terminals,” Report N° 116.

PIANC (2014) “Harbour Approach Channels Design Guideline,” Report N° 121.

Saha, S. et al. (2010) “The NCEP Climate Forecast System Reanalysis.” Bulletin of the American Meteorological Society, August 2010. DOI: 10.1175/2010BAMS3001.1

Senneville, S., St-Onge, S., Dumont, D., Bihan-Poudec, M.-C., Belemaalem, Z., Corriveau, M., Bernatchez, P., Bélanger, S., Tolszczuk-Leclerc, S. and Villeneuve, R. (2014): Rapport final : Modélisation des glaces dans l’estuaire et le golfe du Saint-Laurent dans la perspective des changements climatiques; report prepared by the Institut des sciences de la mer de Rimouski, Université du Québec à Rimouski (UQAR) for the ministère des Transports du Québec., 384 p.

Sherman, K., Belkin, I., Friedland, K.D., O’Reilly, J. and Hyde, K. (2009): Accelerated warming and evergent trends in fisheries biomass yields of the world’s large marine ecosystems; AMBIO: A Journal of the Human Environment, v. 38, no. 4, p. 215–224.

Swail, V.R., V.J. Cardone, M. Ferguson, D.J. Gummer, E.L. Harris, E.A. Orelup and A.T. Cox (2006) The MSC50 Wind and Wave Reanalysis. 9th International Wind and Wave Workshop, September 25-29, 2006, Victoria, B.C.

Walsh, J.E., and C.M. Johnson (1979) An analysis of Arctic sea ice fluctuations, 1953-1977. J. Phys. Ocean., 9, 580-591.

Wang, X. L. and V. R. Swail, (2001) Changes of Extreme Wave Heights in Northern Hemisphere Oceans and Related Atmospheric Circulation Regimes. J. Climate, 14 (10) 2204-2221; DOI: 10.1175/1520-0442(2001)014<2204:COEWHI>2.0.CO;2.

Wang, X. L. et al., (2014) Changes in global ocean wave heights as projected using multi-model CMIP5 simulations, Geophys. Res. Lett., 41, 1026-1034; DOI: 10.1002/2013GL058650.

Wang, X. L. et al., (2015) Historical trends in the Beaufort-Chukchi-Bering Seas surface winds and wave heights, 1970-2013. J. Clim., 28 (19), 7457-7569; DOI: 10.1175/JCLI-D-15-0190.1.

Yin, J. (2012): Century to multi-century sea level rise projections from CMIP5 models; Geophysical Research Letters, v. 39.

Yin, J., Griffies, S.M. and Stouffer, R.J. (2010): Spatial variability of sea level rise in twenty-first century projections; Journal of Climate, v. 23, no. 17, p. 4585–4607.

Young, I.R., Zieger, S., Babanin, A.V. (2011): Global trends in wind speed and wave height; Science, v. 332, p. 451–455.

APPENDIX A: 	HISTORICAL WAVE HEIGHT SUMMARY STATISTICS AND TRENDS

APPENDIX B: 	HISTORICAL EXTREME WAVE HEIGHT INDICES AND TRENDS

APPENDIX C: 	HISTORICAL WIND SPEED SUMMARY STATISTICS AND TRENDS

APPENDIX D: 	HISTORICAL EXTREME WIND SPEED INDICES AND TRENDS

APPENDIX E: 	HISTORICAL MONTHLY WIND DIRECTION

APPENDIX F: 	RCP 4.5 PROJECTED CHANGES IN WAVE HEIGHT SUMMARY STATISTICS

APPENDIX G: 	RCP 4.5 PROJECTED CHANGES IN EXTREME WAVE HEIGHT INDICES

APPENDIX H: 	RCP 4.5 PROJECTED CHANGES IN WIND SPEED SUMMARY STATISTICS

APPENDIX I: 	RCP 4.5 PROJECTED CHANGES IN EXTREME WIND SPEED INDICES

APPENDIX J: 	RCP 4.5 PROJECTED CHANGES IN MONTHLY WIND DIRECTION

APPENDIX K: 	RCP 8.5 PROJECTED CHANGES IN WAVE HEIGHT SUMMARY STATISTICS

APPENDIX L: 	RCP 8.5 PROJECTED CHANGES IN EXTREME WAVE HEIGHT INDICES

APPENDIX M: 	RCP 8.5 PROJECTED CHANGES IN WIND SPEED SUMMARY STATISTICS

APPENDIX N: 	RCP 8.5 PROJECTED CHANGES IN EXTREME WIND SPEED INDICES

APPENDIX O: 	RCP 8.5 PROJECTED CHANGES IN MONTHLY WIND DIRECTION

additional considerations for use include having a large enough average slick thickness (practical), and type, dosage, application techniques and use near specific ecological habitats (regulations requiring approval)